de ON9CSK @ ON7RC.BT.BEL.EU

to KENWOOD @ WW

TH-78A (U.S.A. version) --> TH78E (European version) MODS.

**********************************************************

Hello TH78 user,

If you bought a TH78A in the USA you don't have the 1750 Hz repeater tone

access, but you can change that !

Apart from the well-known six diodes, the new CPU has 2 green wires.

In order to use the TH78A in EU, you must do the following:

- First you have to remove D2 for the standard EU bands (144-146 and

430-440 MHz).

- Now control that the 2 green wires and the 5 other diodes are installed

(D1, D3, D4, D5 and D6).

- If you need band expantion and components location, please see the mods

below.

Although the TH78A has a special bandpass filter for the US 70cm band, I could

not notice a decrease in sensitivity in the EU part of the 70 cm band.

There is a function change for 3 knobs; please see below and also p. 6 and 7 of

your INSTRUCTION MANUAL.

TH-78A   -->   TH-78E

---------------------------

LAMP     -->   TONE 1750 Hz

CALL     -->   LAMP

TONE     -->   CALL

Now, you can choice between the CTCSS tones and the 1750 Hz with the following

sequence: F + LAMP (the new TONE !) and then the right rotary encoder to make

your frequency choice.

*******************************************************************************

KENWOOD TH-78E MODS

*******************

Hello TH-78E user,

I recently bought a TH-78E which apparently had a new CPU. You can see this

internally. Apart from the well-known six diodes it also has two green wires,

indicated on the PCB as W1 and W2.

WARNING !!!

-----------

In the course of performing mods on my Kenwood TH-78E dual band handheld,

I've discovered a potential flaw in the case design. While handling my walkie

one day (after the mods were done), the display went blank and I could not

turn the radio back on. With the radio split in half again, I could turn the

radio back on but discovered all the memories were erased. The cause turned

out to be some component pins on the front face coming in contact with the

square bodies of the two volume/channel/squelch switches, when the case is

screwed back together snuggly. I placed small strips of electrical tape on

the sides of the switches to insulate, and reassembled; problem solved. Now,

I had to reprogram the darn thing....

===============================================================================

The Guide to Undocumented Features of the TH-78E

------------------------------------------------

The information contained in this document is derived from many sources. Thanks

to all. This document is probably not complete; if you have more info's, please

send it to me.

1. Hardware Mods

----------------

Hardware modifications for the TH-78E are performed by manipulating 2 green

wires and a series of 6 surface mounted diodes, identified as D1 through D6.

The TH-78E block diagram contains a table identifying configurations involving

D2, D3, D5, and D6. However, none of the configurations described below are

included in that table.

To open the radio, follow the instructions on p. 64 of the INSTRUCTION MANUAL.

The two TX/RX busy indicators (LEDs) have rubber seals placed over them. These

have a tendency to fall off when opening or closing the radio.

The diodes and wires are located on the CPU board, which is mounted inside the

front panel of the radio. The CPU board is readily identified by a brass shield

covering the CPU, as well as the socket for the ME-1 memory expansion chip.

All position references in this document assume that you are looking at the

CPU board with the rotary encoders and TX/RX busy indicators at the top.

If you have installed the ME-1 EEPROM, I recommend that you temporarily

remove it to facilitate access to the diodes.

The diodes are located in a vertical row just below the lower right corner

of the brass shield. The diodes are numbered sequentially, starting with

D1 at the top.

The E3 configuration (144-146 and 430-440 MHz) has diodes D1, D3, D4, D5 and D6

and the 2 green wires in place.

Remove jumper W1 (one of the two green wires).

RX is: 50-136 AM, 136-180 FM, 300-400 AM/FM, 400-512 FM (VHF SIDE).

50-180 FM, 400-512 FM, 800-1000 FM (UHF SIDE).

TX is: 136-180 FM and 400-512 FM.

Now, you can reprogram the vfo's limits like explained on p.15 of the

INSTRUCTION MANUAL.

Crossband repeat mod: Remove D4.

2. Software Mod

---------------

If you don't need the TX MOD, try the following:

Push PTT + VFO and POWER ON.

TX is not affected, RX is like above.

3. Undocumented Features

------------------------

Crossband Repeater: Press F for 1 second, then 0 (requires D4 mod). Repeat the

same sequence to disable.

Cloning: Press 0 during power-up. Allows cloning over the air. All keys except

PTT and POWER are disabled. Press PTT to send DTMF cloning sequence.

Presumably, receiving TH-78E must be in same mode.

Toggle SHIFT/SPLIT Mode: Press SHIFT during power-up. This is described in

the manual, but the documentation is not complete. The TH-78E can

operate in two modes: In SPLIT mode, non-standard offsets (i.e. split

frequencies) are supported, but the default offset is not programmable.

In SHIFT mode, non-standard offsets are not allowed, but the default

offset is programmable. To select the default offset, press F for 1

second, then SHIFT. See p. 30 of the manual for details on changing the

default offset.

Toggle CALL Button Function: Press CALL during power-up. The CALL button

can operate in one of two modes. In the default mode (CALLSW), it

switches between the call channel and the last memory channel (if in

memory recall mode) or last frequency (if in VFO mode). After toggling

the CALL button functionality (VMC), it will switch from the VFO to the

last memory channel and then back to the CALL channel.

AM/FM switching: Hold down VFO key and switch the POWER ON to switch between AM

and FM reception. This work only in the 300 MHz band !

VHF & UHF expansion: Press the F key for 1 second, then press the BAND key. On

VHF you have now 300-400 AM/FM RX and on UHF 800-1000 FM RX.

Channel display mode: Hold down 3 key and switch the POWER ON. Channels are

displayed by number instead of frequency. (NOTE: this function only works

when Alpha-Numeric memory is enabled).

4. Considerations

-----------------

The display range of the TH-78 is very wide, but usefull range is more little

and is different from one set to the other. For some frequencies, the TH-78

will "beep": this is PLL unlock and there is no RX/TX ! For high frequencies

the red TX led will not glow: no TX ! The locking ranges are different on

MAIN BAND and SUB BAND.

When monitoring 2 VHF freq., the sensitivity of the MAIN VHF BAND is attenuated

but VHF SUB BAND is still good. Thus monitor strong VHF signals on VHF MAIN

BAND and weak VHF signals on VHF SUB BAND. VHF SUB BAND work only in FM and

not in AM.

When monitoring (VHF or UHF) out of ham band signals, sensitivity is realy

better in SUB BAND. Also the rubber duck antenna has only a good efficiency

in the ham frequencies, that's why my preference go to the 37 cm long COMET

CH-600MX antenna.

Sensitivity in the 959 MHz portable telephone band is low, but good enough to

listen the neighbours cordless telephones.

Sensitivity in the 300 MHz band is very low.

When charging PB13 battery, you need 15.5 V on the DC IN TERMINAL to have a

charge current of 70 mA (1/10 of C) for 15 H.

*******************************************************************************

*** GAME *** (Available without any modifications)

--------------------------------------------------

Hold down the M and PTT keys and switch the power ON.

(Be very careful the PTT is depressed or else a memory reset will occur.)

1) Play the memory game.

The radio will beep and display one of the following characters: "1, 2, 3, F"

(top row of buttons). Radio starts with one character and adds a new one on

after every round until 20 characters are reached. You must press the keys in

the same sequence they are displayed. When you can follow the radio for 20

characters, then you will win 100 coins and enter the poker game.

(Note: I am terrible at this game, so I cheated by writing down the character

sequence on paper.)

2) Poker game. (Much more interesting than the first game, IMHO)

Place the bet (1 to 10).

Increase the bet with the up key 2, decrease the bet with the down key 5.

Press F to deal the cards.

Cards are (2,3,4,5,6,7,8,9,+,J,K,Q,A  NOTE: 10 = "+")

Press PTT to see the suits.

Press numbers 1 through 5 to change the respective cards.

Press F to deal the new cards.

If you win any amount, you can bet for double or nothing.

Press F to play "BIG or SMALL".

Press LOW to keep your winnings.

BIG or SMALL

------------

Arandom card will be chosen.

Push 2 (big) if you think this card is going to be larger than 8.

Push 5 (small) if you think this card is going to be less and 8.

(if an 8 is drawn, then a tie occurs, and you neither win nor lose)

Press F to draw the random card.

You either double your winnings or lost it all.

Press F to continue playing Big or Small.

Press LOW to keep winnings and continue playing poker.

Push the LAMP key to exit the game mode.

Note: While in game mode, receive functions still operate!

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Game Mode: Press M and PTT during power-up (be careful not to accidentally

reset the memory (M+POWER)).

To exit the game mode at any time, press the LAMP key. It is not possible

to change the volume, lamp, or frequency settings while in game mode. The

top part of the display will show  H.00, which represents the high score.

The lower part shows a scrolling message, "PRESS ANY KEY". Pushing any key

starts a "Follow Simon" type game. The display will briefly show one of

the characters "1", "2", "3", or "F". Press the corresponding key. The game

consists of repeating the displayed character sequence, which increases by one

character each round.

After you "win" the Simon memory game by getting correctly entering a

sequence of twenty characters, the next game is a draw poker game. The way

it works is that you choose your bet (from 1 to 10) by pressing 2 to

increment the bet and 5 to decrement the bet. Then, press F to deal the

five cards. The face value of the cards is displayed, and the suits can be

seen at any time by holding down the PTT key. Any number of cards may be

discarded, and to select (or deselect) a card for discarding, press the keys

1, 2, 3, 4, or 5. If a card is selected for discard, it is displayed

"face-down".

Press F again to draw new cards. Your new cards will be displayed, and

then if your hand is 2-pair or better, the screen will show the rank of

your hand on the left (2P for 2-pair, 4K for four-of-a-kind, etc.). On the

right the pay- off for that hand will be displayed. Your bet is multiplied

by the pay-off factor,and the resulting pile of cash is displayed in the

right hand side of the upper screen. (The left hand side of the upper screen

contains your table stakes, which are initially 100 coins from winning the

Simon game.)

If you win the poker hand, pressing any key steps into the next stage. If

you lose the poker hand, your bet is deducted from your stakes and you are

asked to start another poker hand. In the next stage, you are asked "TRYB/S"

which means, "Do you want to try double-or-nothing in a guessing game for Big

or Small cards?" Press F for yes, press TONE for no. If you say no, your

winnings are credited into your stakes and you are asked to start another poker

hand. If you say yes, then a single shuffling/incrementing card is displayed

on the left, and three stars are displayed on the right. You have to choose to

go for either BIG or SMALL, by pressing 2 or 5. You can keep pressing 2 and 5

to change your mind. When you are ready, you must try to hit the F key to stop

the rotating card display, and the card will show, and you will either win,

lose, or draw. If you draw, you have to play big/small again, I think. If you

lose, your winnings are gone and you can play poker again. If you win, your

winnings double and you are asked whether you want to play big/small again.

The payoffs on the poker are set against you, odds-wise; the double-or-nothing

game includes a draw, so the odds are against the player there unless you can

time hitting the F key to win more than half the rounds. I haven't managed to

do this, so I don't know if there is anything beyond this, all I know is that

when the table stakes are exhausted, you go back to playing Simon again.

*******************************************************************************

73's and try it at your own risk ! I haven't tried the games.

Karl, ON9CSK @ ON1BWP.#BTN.BEL.EU