Doc for CAT_SEED.BAS/.TXT                                 K5YEF  Page 1

A GW-BASIC APPROACH TO CAT CONTROL - A 'UNIVERSAL' SEED

by Art Harding K5YEF

(Partial reprint from 73 Magazine, August '91. The magazine

has exercised their first publishing rights so this is now

supplied by the author in the Public Domain.  For reprints of

the original article send $3.00 to 73 Magazine / WGE Center /

Forest Rd. / Hancock, NH 03449 - ask for UNIVERSAL CAT

INTERFACE, Aug. '91)

After you do the simple hardware tests on the Universal CAT Interface,

you'll be itching to try your system in the CAT mode.

As author of the pascal program CAT980 which controls the YAESU(tm) FT-

980 transceiver, I have come into contact with a lot CAT enthusiasts

since the offering of the program in 1988.  Many of them owned radios

other than the FT-980 but they were hoping against hope that somehow

CAT980 could be adopted to their shack. Sorry, but no way.

The big problem of course is the lack of a CAT standard. If there are

two radios with the exact same I/O command set I am unaware of them. I

would urge the radio industry to get together in some kind of a summit

meeting. Agree on a command set for CAT control. Don't be part of the

problem, OEM's, become part of the solution. (And hey, how about

defining "S9" while you're at it!)

So I understand the frustration of those who have attempted to write

their own program or to adapt existing programs to their radio.  Early

on there were times when I think I would rather have been in DX pile up

with one watt then to watch a dead screen or worse, a crashed system.

What we're dealing with here is one big loop from computer to interface,

from interface to radio, from radio to interface, and from interface to

computer. And if anything, anything at all is wrong in the software or

the hardware then you might as well try working the spirit world when

the band's dead. No pun intended.

Presented here is a GW-BASIC (Version 3.2) program which I believe you

can use to adapt to any CAT equipped radio. "Those are big words,

stranger." I know, I know. But you have to start somewhere and I would

like to plant this program which I have put into the Public Domain into

your schedule and let you run with it. (Pun is intended.)

It's not going to be easy but it is simple.  You'll understand what I

mean if you'll commit to the attempt, and if you will just maintain an

I-CAN-DO-IT attitude, take your time and keep trying, you will

eventually get the desired results. And I can't describe what a great

feeling it is to see the radio respond to your keyboard.

You lucky FT-980 owners can use the program as is.  It will come up and

capture the radio and display all 148 status bytes from the radio. It

will settle on 20 meters, USB, 14.250.000, picked because that is the

example in the YAESU manual. Hit <ENTER> and the radio switches to AM

and WWV on 10 MHz. Hit <ENTER> again and the radio switches to 10 meter

Doc for CAT_SEED.BAS/.TXT                                 K5YEF  Page 2

BASIC CAT Pgm Seed

FM and begins a frequency scan from 29.50 to 29.690 ad infinitum. You

can interrupt the scan or resume it by hitting any key at any time. The

screen displays the frequency too. <F1> exits the program at any point

you desire.

The one quirk 980 owners will discover, and its with the radio, is that

it will require two tries to capture the radio the first time. Run the

program and after a few seconds do a Control Break. Then run it again.

From then on (unless you turn the radio off) it will run on the first

try. Sure, you can add a fix. I had to add one to my pascal version but

it is beyond the scope of this exercise.

OK, so what if you own a radio other than the FT-980 which is most

likely the case? This program will be a good solid starting point. Oh

yes, it will require some knowledge of BASIC, patience and study, but

the trek will be worth it. The program has the two main ingredients

necessary for CAT activity: a way to output commands and a way to

capture data from the radio.

First study your manual and become familiar with the sequencing of

events that your radio requires. The FT-980, for example, requires the

following:

1. Send a command to the radio.

2. Receive an echo of the command from the radio.

3. Have your software compare the command against the echo. If

they are the same then perform the next step, #4. If not

then start over again at step #1.

4. Send OK-TO-EXECUTE command to the radio.

5. Receive a status stream from the radio to update the

changes it just made.

WOW! Well the fact is the FT-980 is one of the more complicated CAT

radios ever put on the market. (I give that a plus, not a minus; get you

one!) Unlike the sequence listed above, your radio may only need to

receive a command to change mode or frequency. It may or may not 'talk'

back to your computer at all. So at this point we can begin to trim down

the CAT_SEED program to customize it to your radio. In the following

steps when you're advised to 'eliminate the following lines' you might

wish to simply REMark them out - just in case.

TAILOR THE PROGRAM FOR YOUR PARTICULAR RADIO

--------------------------------------------

1. Eliminate line 2030; it surely just applies to the FT-980.

2. If your radio isn't captured during a CAT session, that is

if it only responds to commands but the front panel controls remain

operational between commands, then you should eliminate the following

lines: 90, 1070 and 2020.

2

Doc for CAT_SEED.BAS/.TXT                                 K5YEF  Page 3

BASIC CAT Pgm Seed

3. If your radio does NOT echo the commands you send it you

should eliminate the following lines: 1020, 6050 thru 6080.

4.If your radio does NOT have an OK or EXECUTE-THE-COMMAND

type command you should eliminate the following lines: 100, 1060, 5030,

7000 thru 7030.

5. Does your radio send back any information like a status

stream or some sort of acknowledgement that the command took effect? If

your radio does NOT send back anything you should eliminate lines: 1030,

1080, 2110, 3030, 5040, 6030, 8000 thru 8040, 9100 thru 9150, and 10000

thru 10020. You may eliminate the variable R and all references to R.

Now the next thing you want to do is study the INITIALIZATION portion of

the program, lines 1000 thru 1130. Whatever brand of radio you own it's

going to need a command buffer defined in line 1040. The one in our seed

program is presently 5 bytes. If you need less or more then change it

accordingly. A 6 byte command would appear as CMD$="      " (that was

six spaces) or perhaps even better, use DIM CMD$(6).

Other FT-980 commands are part of the initialization subroutine such as

the command for VFO, USB, etc. Simply substitute commands required by

your radio. If there is no substitution in some cases then eliminate

that particular one and any other reference to it you may find in the

program. After you get this program working then you can add the

complete command set.

If your radio does receive status then be sure line 1030 is configured

correctly. It is now 148 bytes, the maximum number of bytes the FT-980

radio will send to the computer at any one time.  You should make it as

large as the number of bytes your manual shows is the maximum you may

receive from your radio.

What's left? One of the most important things: setting up the COM port

for your particular needs. Let's look at line 9010:

OPEN "COM1:4800,N,8,2,RS,CS10,DS,BIN" FOR RANDOM AS #1

Check your GW-BASIC manual and your radio manual; make sure these

parameters are correct. More than one program I've seen was 'bad'

because the author used one stop bit. Why not? The rest of the world

uses one stop bit - but not the FT-980 PROM. It demands two! So make

sure all is OK. The parameter 10 associated with CS above will most

likely have to drop to 0 if you are using a slow computer and a value

higher then 10 for faster machines. I have tested the program at 4.77

MHz on a lap top (CS0) and at 16 MHz on a 386 machine (CS10).  One other

comment on the port: you may also use COM2 instead of COM1. If you wish

to do this be sure and change all COM references throughout the program.

(AND - if you still have problems 1)reboot 2)run FIX_COMS.EXE 3)try the

above again!)

3

Doc for CAT_SEED.BAS/.TXT                                 K5YEF  Page 4

BASIC CAT Pgm Seed

As you work with the program I recommend that you just concentrate on

getting the computer/radio dialog going with consistent results. Then

you can open up your imagination and add the bells and whistles. Today,

CAT980 has some pretty fancy displays like filter plots, analog dials

and user menus for instant deployment. Even a log is included which

automatically records all the radio parameters of the moment. But there

was a time when the program  just sat there and did nothing. Hi hi.

Did I miss anything?  Where any lines removed that should have stayed

put or have lines remained that should be long gone?  I don't know, I

really don't.  This is a mental exercise; only you can take it from the

hypothetical to an empirical happening.  But we're all hams, we're

experimenters and developers. Our hobby is where it is today because we

can't leave well enough alone.  Thank goodness.

So with that in mind here is one last thought: share your basic ideas

with other hams. Write an article or submit a listing to the technical

correspondence sections of our magazines. Put your program on BBS's like

CompuServe. And if you feel like you've got a program for the big

leagues, then enter the world of Share Ware.  The real fun, and I

believe this strongly, is still out there.  Share!

K5YEF / CAT980 Ver 1.9 / P.O. BOX 1719 / Plano, Tx. 75074 / USA

4