RITTY 2.0

Copyright 1997 by Brian Beezley, K6STI

All Rights Reserved

RITTY 2.0......................................1

Evaluation Version.............................1

Use in Windows.................................1

Sound Card.....................................1

FSK, PTT, and TTY Output.......................3

Displays.......................................4

Using RITTY....................................4

Signal Processing..............................6

Setup Menu.....................................9

Receive Options Menu..........................12

Transmit Options Menu.........................14

Baudot Punctuation............................16

Screen Colors.................................17

Implementation Notes..........................17

Use with RTTY by WF1B.........................18

Copyright and License.........................21

---- RITTY 2.0 -------------------------------------------------

RITTY combines a high-performance FSK modem with a

terminal program to enable you to transmit and receive

radioteletype and PACTOR signals on your PC.  RITTY uses

advanced digital signal processing algorithms but does not

require DSP hardware.  RITTY uses your sound card for analog

input/output and optimized assembly language code for speed so

it can run right in your PC.  RITTY requires a 486 or better,

math coprocessor, VGA, sound card, and DOS 3.1 or later.

This document describes general aspects of RITTY and

RTTY mode.  See PACTOR.DOC for information about PACTOR mode.

---- EVALUATION VERSION ----------------------------------------

For full functionality, put your RITTY.LIC license file

in the directory that contains RITTY.EXE.  The unlicensed

evaluation version cannot TSR, runs for a limited time, and does

not retain its state when restarted.

---- USE IN WINDOWS --------------------------------------------

The following instructions assume that all RITTY files

are in \RITTY.

To install RITTY in Windows 3.x, select a program group

for RITTY in Program Manager.  Select File | New.  In the New

Program Object menu, select Program Item and click OK.  In the

Program Item Properties menu, under Description enter RITTY.

Under Command Line enter RITTY.  Under Working Directory enter

\RITTY.  Under Shortcut Key enter R.  Click Change Icon.  Click

OK when told that no icons are available.  In the Change Icon

menu, under File Name enter \RITTY\RITTY.ICO.  Click OK

repeatedly to dissolve all menus.

To install RITTY in Windows 95, start Explorer and

navigate to \RITTY.  Using the right mouse button, drag the

RITTY.EXE icon to the desktop.  Click on Create Shortcut(s)

Here.  Next, right-click on the dragged icon.  Click Properties,

then click the Program tab.  Click on Change icon.  Click

Browse, navigate to \RITTY, and double-click on RITTY.ICO.

Click OK twice.

To run RITTY, double-click on its icon.  Press Alt-Tab

to switch to another program.  To switch back to RITTY in

Windows 3.x, press Ctrl-Alt-R.  (Switching to another program in

Windows 3.x may disrupt copy).

---- SOUND CARD ------------------------------------------------

RITTY works in RTTY mode with most sound cards, but

PACTOR requires a genuine, 16-bit Creative Labs card.  Other

1

cards also may provide only coarse gain adjustment and exhibit

limited dynamic range.

RITTY uses the BLASTER environment variable to determine

sound-card hardware settings.  When BLASTER is missing RITTY

reads the settings from 16-bit Creative Labs cards and assumes

factory-default interrupt and DMA settings for others.  You may

need to program the settings to any card on power-up with a

utility like DIAGNOSE.  See your sound-card manual for details.

RITTY does not require sound-card device drivers.

You can obtain receive audio from your transceiver's

speaker output, but it's better to use a fixed-level audio

output when available.  This lets you adjust AF gain for

comfortable listening without affecting RITTY.  Use fast AGC and

set RF gain to maximum.  Connect receive audio to the sound-card

left- or right-channel line input.  If your receiver can't fully

drive the line input, use the mic input instead.

NOTE:  It's easy to overdrive the sound card's high-gain

microphone preamp.  Reducing the input-gain setting won't help

because the gain-control stage comes after the preamp.

Therefore, it's important to check for distortion whenever you

use the mic input.  Tune your receiver past a strong, clean sine

wave.  Look for harmonics generated by an overdriven preamp in

the spectral display.  You should see just one signal pip as you

tune through the SSB passband.  If your receiver's fixed-level

output overdrives the mic input, add a series resistor (try 3.3k

ohms) to form a voltage divider with the 540-ohm mic input

resistance.

Adjust input gain in 2-dB steps using the Setup menu.

(Many sound cards have coarser steps and limited adjustment

range.  Older cards may provide no gain adjustment at all).  The

bar graph in the lower-center of the screen depicts peak input

level on a linear scale.  When the graph turns red, the A/D

converter has clipped.  16-bit Creative Labs sound cards provide

so much dynamic range there's no reason to clip the input.

Raise the input level until clipping occurs.  Then back it off

one step.  Infrequent, brief clips won't affect copy, so don't

worry if the graph occasionally flashes red.

You can connect the mic, line-left, and line-right

inputs to separate sources.  Each input maintains its own input-

level setting so you can switch among the sources without

readjusting input level.  (Some sound cards may not provide

source selection.)

RITTY can provide AFSK or FSK output.  See the next

section for FSK.  RITTY generates AFSK on the left channel.

Route the left-channel line output (mini phone-plug tip) to your

transceiver's packet, phone-patch, or mic input.  VOX works fine

to switch between transmit and receive on RTTY.  You can use the

push-to-talk (PTT) line instead if you build the interface

circuit described in the next section.  PACTOR requires PTT

keying.

2

To set AFSK transmit level, temporarily enter 100% for

Rated TX Duty Cycle in the Setup menu.  Switch your transceiver

to LSB.  Turn off your speech processor and disconnect your

microphone if active.  Set your mic gain to the position you

normally use for voice.  Set your power-output control to

maximum.  Press Pause to transmit.  Adjust Output Level in the

Setup menu to the point where your ALC just begins to activate.

(You can slightly alter mic gain for a more precise setting.)

To use VOX, enable it, set anti-VOX and VOX delay to minimum,

and adjust VOX sensitivity so that your rig keys reliably when

you press Pause.  Finally, enter your transmitter's duty-cycle

rating for extended, key-down transmissions.  RITTY will

automatically adjust AFSK output to the proper level for all

transmit modes.  If you use FSK or don't operate PACTOR, leave

the duty-cycle parameter set at 100%.

If you have a wattmeter, it's simpler to just enter the

rated duty cycle, press Pause, and adjust Output Level for

maximum rated RTTY or FM output.  To avoid clipping your mic

preamp, make sure your mic-gain control is set to its normal

voice position.

Carefully check your transmit signal for hum and noise.

If present, add a 10k-ohm resistor in series with the line

output and a 330-ohm resistor across your transceiver's audio

input.  Then reset transmit audio level.

RITTY generates phase-continuous, 16-bit, sine-wave

output (8-bit output when not using a 16-bit Creative Labs sound

card).  The output signal is cleaner and more accurate than the

FSK provided by many transceivers.  (For example, the Kenwood

TS-930S and TS-940S generate 165.9-Hz FSK by lowpass-filtering

the square-wave output of a digital divider and inserting this

signal into the SSB transmit audio chain.  This really is AFSK!)

---- FSK, PTT, AND TTY OUTPUT ----------------------------------

Some transceivers won't let you use a CW filter in LSB

mode but allow it in FSK.  To permit use of a narrow IF filter,

RITTY can generate FSK output on the transmit-data pin of a COM

port.  Select the port in the Setup menu.  Mark is a negative

output voltage and space is positive.  You may be able to apply

the RS-232 signal directly to your rig's FSK input.  If not, use

an interface circuit like that described next for the PTT

signal.  (To invert FSK polarity, swap the base and emitter

leads.)

RITTY drives both RTS and DTR of the selected COM port

with a PTT signal.  Transmit is a positive voltage, receive is

negative.  You can obtain PTT from a parallel port instead (AFSK

only).  RITTY drives all eight TTL outputs high for transmit and

low for receive.  For parallel-port output select COM port X in

the Setup menu.  For Port X Address enter 378 for LPT1, 278 for

LPT2, or 3BC for LPT3.  Connect a 1k-ohm resistor from the RS-

3

232 or TTL output pin to the base of an NPN transistor.  Ground

the emitter and connect the collector to your PTT line.

RITTY provides Baudot output on the COM port during

receive for hardcopy on a Teletype machine.  RITTY outputs at

most 72 characters per line, pads the data stream to provide

sufficient time for mechanical carriage return, inhibits

overprint, suppresses diddle for quieter operation, and ignores

isolated LFs when enabled.  Engage your Teletype's unshift-on-

space mechanism to enable that feature.  RITTY disables TTY

output when TSR.

---- DISPLAYS --------------------------------------------------

RITTY runs your VGA in 640 x 480 graphics mode.  This

allows it to display 30 lines of text using custom fonts with

special symbols.  It also permits real-time signal display.

The graph in the lower-left screen corner is a spectral

tuning indicator.  The yellow trace is a fast Fourier transform

(FFT) of the input signal.  The cyan trace is the average

spectrum.  The purple lines show the receive-filter locations.

The height of each line is proportional to filter output.  Tune

your receiver to center the signal pips on the purple lines.

The red bars below the baseline indicate the AFSK transmit

frequencies.  Use the Setup menu to select FSK center frequency,

the up- and down-arrow keys to change frequency shift (+ and -

also work), and the left- and right-arrow keys to fine-tune the

shift.

The lower-right screen corner displays one of three

waveforms.  The character waveform shows the demodulated mark-

minus-space signal for one character length.  From left to right

the red tic marks locate sample points for the start pulse, the

five Baudot data bits, and the stop pulse.  The D1 display is a

sliding window of several seconds of demodulated data and the

detection threshold.  The yellow dots are the data samples from

the character display.  Above the cyan trace a data point is

detected as a mark and below as a space.  D1 reveals the effects

of selective fading, flat fading, static crashes, automatic

threshold correction, etc.  The D2 display is a stationary

version of D1.

---- USING RITTY -----------------------------------------------

RITTY maintains its state between sessions (licensed

version only).  When you quit, RITTY saves all menu settings and

messages in RITTY.INI in the directory containing RITTY.EXE.  If

RITTY.INI exists when you start the program, RITTY reads the

file and reinitializes itself. Otherwise, it uses default

settings.  Delete RITTY.INI to return to factory defaults.

During receive, press any unused key to display a list

of commands and functions.  A menu appears when you press a

4

command key.  Exit the menu by pressing Esc.  You also can exit

by pressing the command key a second time.  This is handy when

making a quick check.  You can go directly from one menu to

another by pressing a command key.

To change a menu item, use the up- and down-arrow keys

to position the red pointer to the item.  If the item offers a

choice, use the left- and right-arrow keys to move the highlight

to another selection.  If the item is numerical, type in a new

value.  After typing all digits, press Esc to enter the item and

dissolve the menu.  The next time you bring up the menu, the

cursor will be where you left it.  Terminating numerical entry

with Esc is handy for repeatedly changing a single parameter to

observe its effect.  Press Enter instead of Esc to retain the

menu and advance the cursor to the next item.  Terminating entry

with Enter is good for changing several items at once.

Several menu items have hotkeys.  You can alter one of

these parameters without accessing a menu by simultaneously

pressing the Alt key and the hotkey.  (Hotkeys also work when

the menu is visible.)  Hotkeys are underlined in the menus.

Press Pause to enter transmit mode.  RITTY transmits

whatever you type.  Press Pause or Esc to return to receive.

You can insert a message into the transmit character stream by

pressing F1 through F12.  Prepare these messages in receive

mode.  (You can edit a message while transmitting by pressing

Alt-F1 to Alt-F12.)  Each message can be up to 28 lines long.

When entering a message, use any of the editing keys listed,

plus Backspace and Del.  Alt-W deletes a word, Alt-D deletes a

line, and Alt-C clears the entire message.  Press ^right-arrow

or ^left-arrow to skip to the next or previous word (^ means the

Ctrl key).  Press ^R to enter a box symbol.  When RITTY

encounters a box during transmit, it switches back to receive

mode.  You can tack a box onto the end of a long CQ before

wandering into the kitchen to grab a beer.  If the CQ ends

before you return, your transmitter will automatically shut off.

You can also enter ^R in transmit mode when typing ahead.

Use any of the message buffers to begin replying to a

station while he's sending.  If he turns it over to you while

you're still typing, don't bother to exit the message editor.

Simply press Pause.  The message will be inserted in the

transmit buffer and you'll enter transmit mode.  You can finish

typing the message as it's being transmitted.

To transmit an ASCII file, enter the filespec (filename

preceded by directories, if necessary) in one of the message

buffers.  RITTY will transmit the file contents instead of the

filespec.  RITTY sends BLANK for ASCII characters without Baudot

equivalents.

Press Esc to erase the transmit buffer.  This is handy

after pressing the wrong function key.  Press Alt-M or Alt-S to

output a steady mark or space tone.  This is useful for

5

measuring frequency or checking the amplitude ripple of your

transmit-IF filter.

The only editing key available in transmit mode is

Backspace; it works only on the current line.  You can backspace

to erase any character not yet transmitted (colored white).  You

can enable whole-word output to facilitate typo correction.  See

the Transmit Options Menu section.

Except when TSR, RITTY automatically records all receive

and transmit text to a file named TEXT in the current directory.

After exiting RITTY you can edit TEXT to extract desired

information.  RITTY overwrites TEXT each time it executes.  Even

unsquelched, TEXT grows only by 21 kB per hour at 45 baud.

Unless you let RITTY run indefinitely, you'll never need to

worry about filling up your hard disk.

---- SIGNAL PROCESSING -----------------------------------------

RITTY's purpose is to recover RTTY signals other modems

can't detect.  Here's how it works:

Input BPF

RITTY provides a 200-tap, linear-phase, FIR input

bandpass filter.  Use this filter for QRM when you lack a CW

filter or yours is too broad.  The BPF is 280 Hz wide at -6 dB,

425 Hz wide at -60 dB, has a double-humped response with a 16-dB

central depression (each hump is 100 Hz wide), and provides 54

dB of ultimate rejection.  Enable the filter with the Receive

Options menu.  It autotunes along with the channel filters as

described below.  The BPF slightly degrades receive sensitivity

when only noise is present.

When you enable the input filter above 45 baud, RITTY

uses the PACTOR BPF instead (500 Hz at -6 dB, 800 Hz at -60 dB,

and ultimate rejection > 100 dB).  No filter is enabled for 425-

or 850-Hz shift.

Narrowband Detector

The narrowband detector maintains signal linearity as

long as possible.  It doesn't use an input limiter.  This allows

it to extract signals buried in noise and QRM without suffering

the limiter capture effect.

The narrowband detector uses a pair of matched filters,

one each for mark and space.  The filters have the SIN(x)/x

shape optimal for the detection of tone bursts in additive,

white, Gaussian noise.  The filter widths track baud rate and

the center frequencies are adjustable.

6

The matched filters feed an I-Q envelope detector.

RITTY does not do post-detection, low-pass filtering.  Any

filtering beyond that provided by the matched filters would

degrade sensitivity.

The matched-filter peaks are quite narrow at the slower

baud rates; tuning is critical.  Try to tune signals within 10

Hz.  Although the peaks are narrow, the filter skirts drop off

slowly.  Strong signals in the skirts can disrupt copy.  Use a

500-Hz CW filter or RITTY's BPF when QRM is heavy.  (To use an

external DSP filter, see DSPFLTR.DOC.)  Even without a filter,

the narrowband detector is much less sensitive to QRM than

ordinary detectors because it doesn't use an input limiter.

Wideband Detector

Although the narrowband detector maximizes sensitivity,

it does not respond well to signals corrupted by severe

ionospheric disturbance.  For example, polar flutter can

modulate a signal with AM and FM components.  Both components

can create incidental sidebands that broaden the signal spectrum

beyond the matched-filter passbands.  In addition, the FM

component can degrade phase coherence.

RITTY provides a wideband detector that responds to all

available signal power.  The detector (a zero-crossing counter

with a rectangular, bit-length, running window) has a very short

coherence time.  Signal phase can wildly fluctuate during a bit

period without significantly affecting detector output.

The wideband detector implicitly contains a limiter and

therefore benefits from prefiltering.  RITTY enables the RTTY

BPF (200 Hz noise bandwidth) along with the wideband detector by

default.  Although unlikely to help, you can try the 500-Hz

PACTOR BPF for severely corrupted signals (this filter lowers

receive sensitivity another 4 dB).  You can disable the BPF for

slow computers.  The BPF (or a CW filter) improves the

detector's sensitivity and reduces its susceptibility to QRM.

When you switch back to the narrowband detector, RITTY restores

its BPF setting.  The wideband detector doesn't require ATC;

autotune provides this function.

Since the wideband detector doesn't use channel filters,

RITTY instead draws a dotted line in the spectral display at the

frequency corresponding to the zero-crossing count that

differentiates mark and space.  The granularity in the waveform

display is due to the small number of zero-crossings during a

bit period.

ATC

RITTY provides a special automatic threshold correction

algorithm for use with the narrowband detector.  ATC modifies

the threshold that distinguishes mark from space.  The

7

unmodified threshold is zero (mark and space are weighted

equally).  ATC alters the threshold so that channel-amplitude

differences due to selective fading, IF-filter ripple, and audio

roll-off are compensated.  In effect, ATC processes the channels

independently to provide frequency-diversity reception.  This is

a powerful technique for recovering fading signals.  In some

cases ATC can provide perfect print for signals that otherwise

would be completely garbled.  For example, ATC lets you notch

out QRM in one channel with your receiver's notch filter and

maintain solid copy using the remaining channel.  ATC really

helps fight HF propagation anomalies and interference.

RITTY's ATC algorithm peeks into the future to examine

upcoming signal characteristics so it can determine an optimal

threshold for detecting the current character.  Display the D1

or D2 waveforms to view ATC action.  For general reception set

ATC to Auto in the Receive Options menu.  This lets ATC

continuously vary between fully off and fully on as conditions

warrant.  Full ATC may degrade copy for unstable signals

corrupted by polar flutter or static crashes, or for weak

signals buried in noise.

Autotune

RITTY provides automatic receive-filter tuning.  When

enabled, the filters quickly lock to the strongest tones within

the autotune passband and then slowly track them.  When using

narrow shift, autotune accepts shifts between 135 and 250 Hz.

Control autotune and its passband from the Receive Options menu.

Press the spacebar to freeze the filter frequencies.  The filter

markers turn gray when frozen.  Press again to unfreeze.  QRM

may capture the filters, but you can cycle through the three

most recent filter settings by pressing Tab.  Tab also freezes

autotune.

Autotune modifies the zero-crossing threshold count for

the wideband detector.  Effectively this provides ATC.  In

addition, autotune facilitates reliable wide-detector squelch.

Use the Transmit Options menu to enable AFSK autotune.

This allows the AFSK center frequency to track that of the

received signal.  For narrow shift, the transmit shift will be

170 Hz for received shifts less than 185 Hz; otherwise it will

be 200 Hz.

Numerical Flywheel

RITTY tries to lock a numerical flywheel to the incoming

data stream.  The flywheel synchronizes its rotation to the

received-character timing.  When locked, the flywheel provides

precise, regular timing for data samples that's much less noisy

than that derived from raw start pulses.  The numerical flywheel

can maintain lock on weak signals and on strong signals through

deep fades.  It provides the precise synchronization necessary

8

to avoid intersymbol interference and to fully exploit the

processing gain of the matched filters.  The numerical flywheel

essentially lets you synchronously sample asynchronous RTTY.

The numerical flywheel is useless if the transmit

station emits characters at an irregular rate.  This nearly

always occurs for hand typing without diddle, and it's

characteristic of some older RTTY equipment even with diddle

enabled.  For example, Kantronics KAM multimode controllers with

older firmware transmit most characters with two stop bits but

occasionally insert one with 2.5.  Vintage Tono terminal units

output diddle at half the normal character rate.  These peculiar

signals will cause the numerical flywheel to lose lock.  RITTY

detects irregular character emission and disables the flywheel,

but not before garbling a few characters.  It tries to relock

after any disturbance, so the cycle will repeat.  Use the

Receive Options menu to disable the numerical flywheel for

signals with irregular timing.  Better still, select Auto for

general use.  In this mode the numerical flywheel is much less

likely to lock to irregular signals.  When it does, it nearly

always will disable itself without losing a character.

When the numerical flywheel is locked, RITTY displays

character length below the waveform display.  You'll notice a

wide variety of lengths transmitted on the air.  The standard

length for 45-baud RTTY with a 31-ms stop bit is 163 ms.  You'll

see signals with all the way from one stop bit (154 ms), to two

(176 ms), and even beyond.  The numerical flywheel can lock to

bit streams that use from 0.75 to 2.25 stop bits.

Squelch

RITTY provides a squelch to inhibit copy when there's no

signal.  You can select off, loose, or tight.  When the squelch

suppresses one or more characters, RITTY displays a red dot.

RITTY overwrites the dot with a new character when the squelch

opens.  If red dots momentarily appear during a transmission, it

means you're missing text; lower the squelch threshold to

recover more.

---- SETUP MENU ------------------------------------------------

Source

Select the mic input only if your receiver can't

adequately drive a line input.  Be sure to check for sound-card

distortion when using the mic input.  See the Sound Card section

for details.  (Older sound cards may provide no input-source

selection.)

9

Text Font

This parameter determines the display font for receive,

transmit, and edit text.  Sans means sans serif.

Zero Style

RITTY provides plain, dotted, and slashed typefaces for

the digit zero.  Select the style you prefer.  Note that a plain

zero and the letter O have different shapes.

Audible Bell

Select Yes to have RITTY emit a short beep when it

receives a bell code.  Select No to display a cute little bell

symbol instead.

WF1B Det Key

Alt-~ in RTTY by WF1B normally toggles between narrow

and wide detectors.  Select - or \ if your keyboard doesn't have

the ~ key.

Input Level

Gain resolution is 2 dB for 16-bit Creative Labs sound

cards.  RITTY rounds down odd-numbered entries.  (Other sound

cards have coarser resolution and limited gain-adjustment range.

Older cards may provide no input-gain adjustment at all.)  Each

source maintains its own input-level setting.

Output Level

Gain resolution is 2 dB for 16-bit Creative Labs sound

cards.  RITTY rounds down odd-numbered entries.  (Other sound

cards have coarser resolution and limited gain-adjustment

range.)

Output Delay

RITTY can delay transmit output to give your transceiver

and amplifier time to switch.  This prevents text loss.  (For

AFSK/PTT it also prevents hot-switching your relays and inhibits

ALC spikes caused by briefly transmitting into an open circuit.)

You can enter a delay of up to 99 milliseconds.  For AFSK/VOX

and FSK, RITTY implements a nonzero delay by inserting one

diddle character.  For AFSK/PTT and PACTOR, RITTY delays the

exact amount specified.

10

Output

When you select FSK you must select a COM port for

output data.  The selected port also provides PTT for both FSK

and AFSK.  FSK output bits can't be software-inverted and

there's no way for FSK shift to track receive shift.

COM Port

Select the COM port for FSK and PTT output.  Select

Unused for AFSK/VOX or X for a nonstandard port.

Aux Port

This auxiliary COM port provides timing for PACTOR FSK.

It cannot be set to the COM port used for FSK output.  Select

Unused for AFSK or Y for a nonstandard port.

Port X Address

Specify a nonstandard hex address for the COM port.  For

PTT output on a parallel port (AFSK only), enter 378 for LPT1,

278 for LPT2, or 3BC for LPT3.

Port Y Address

Specify a nonstandard hex address for the Aux port.

Port Y IRQ

Specify the IRQ for a nonstandard Aux port.

Center Freq

Center frequency is halfway between the mark and space

tones.  RITTY's performance is not affected by the choice of

center frequency.  It's best to set it to coincide with the

passband of your CW filter.  You might also set it for lower-

frequency tones that are more pleasant to listen to.

RTTY Baud

You can select one of the following transmission speeds:

Speed           Element Length

45 baud              22 ms

50                   20

57                   17.6

11

75                   13-1/3

100                   10

Rated TX Duty Cycle

Enter your transmitter's percentage-of-full-output

rating for extended key-down transmissions.  For example, if

your 100-watt transceiver is rated at 50 watts output for RTTY

or FM, enter 50%.  RITTY then will automatically adjust AFSK

output to the proper level for all transmit modes (assuming

PACTOR control signals just hit maximum short-term output).

---- RECEIVE OPTIONS MENU --------------------------------------

Flywheel

Select Auto for general use.  Use Off for the rare

signal with timing too irregular for Auto to handle.  Select On

for best copy of regular signals through slow, deep fades or

those buried in noise.

ATC

Select Auto to let ATC automatically and continuously

vary between fully off and fully on as conditions warrant.

Select Off to disable ATC.  Select On to fully engage ATC when

you believe Auto is too conservative.  The length of the solid

part of the character-waveform display baseline varies between

zero and full as ATC varies between fully off and fully on.

Detector

Select Narrow for normal use.  Use Wide for signals

corrupted by severe polar flutter.  The narrowband detector

provides maximum sensitivity.

Wide Det BPF

This setting determines the BPF used with the wideband

detector.  Select 200 to maximize sensitivity.  Select 500 for

severely corrupted signals (this reduces sensitivity 4 dB more).

Select Off when your computer is too slow to run the BPF.

BPF

Enable the input bandpass filter for heavy QRM.  The

filter autotunes along with the channel filters.  It

automatically disables itself at 425- or 850-Hz shift.  The BPF

requires much computation and may not run on slower computers.

12

Autotune

Select On to let the channel filters automatically tune

to the received-signal tones.  Press the spacebar to freeze the

filter frequencies and Tab to cycle through the three most

recent filter settings.

Autotune Width

Use this parameter to narrow the width of the autotune

window to help keep QRM from capturing the filters.

Unshift on Space

Unshift-on-space automatically reverts the decoder to

LTRS mode whenever a space is received.  This helps prevent

printing long strings of wrong-case text.  However, some modems

and press broadcasts don't anticipate the use of this feature

and will print incorrectly.  Disable unshift-on-space for these

signals.

On transmit, RITTY assumes that unshift-on-space may be

used at the receiver.  Therefore, in FIGS mode it always sends

LTRS or FIGS before the first nonspace character following a

space.  This procedure provides compatibility with all decoders.

(LTRS is the Baudot code that causes subsequent characters to be

interpreted as letters.  FIGS is the code that shifts the

decoder to numbers and punctuation.)

Ignore Isolated LF

RITTY can ignore LF codes that don't occur in the first

print column.  This helps to keep more text on your screen when

copying noisy signals.  LF is seldom (if ever) transmitted

except in the first column.  (LF stands for line feed.)

Display Both Cases

RITTY can simultaneously print an alternate character

stream that uses the other LTRS/FIGS sense.  This feature is

really handy when DXing weak and fading signals that may get

stuck in the wrong case.  You'll find the correct shift sense on

one of the two lines displayed.  Often this will give you the

crucial, missing part of an incomplete callsign.

When you're not displaying both cases, you can recover

from shift errors by looking northwest from the QWERTY...

keyboard row to identify corresponding digits.

13

Show Control Codes

RITTY can print symbols for the CR, LF, LTRS, FIGS, and

BLANK control codes.  This can reveal interesting quirks and

properties of various modems.  It also can show the redundancy

employed by some stations to combat noise (e.g., W1AW

bulletins).  (CR stands for carriage return.  BLANK is a

nonfunctional code.)

Tones

The standard for amateur RTTY FSK is that the RF

frequency for mark is higher than that for space.  Nearly all

commercial RTTY signals use the opposite convention.  Selecting

Reverse is more convenient than switching to USB and retuning.

The waveform display does not invert when you reverse the tones.

Squelch

Loose squelch inhibits most noise characters but still

provides copy of weak signals.  Select Tight when signals are

good.  Turn the squelch off for very noisy signals or those with

deep QSB.  The tight-squelch threshold is 3 dB higher than that

for loose.  In addition, the start pulse of each character must

be a space and the stop pulse a mark.

Waveform

Select Character for a high-resolution, character-length

display of the demodulated mark-minus-space signal.  Select D1

or D2 to view the signal envelope and ATC action.

---- TRANSMIT OPTIONS MENU -------------------------------------

Word Output

RITTY can transmit individual characters as soon as you

type them or it can transmit a complete word after you've

followed it with a space, Enter, or ^R.  This provides a chance

to correct a typo when you're not typing far ahead.  (After

entering a space or ^R, backspacing reinhibits word output to

provide yet another chance.)

Duplicate LTRS FIGS

RITTY can add transmit redundancy by sending each LTRS

and FIGS twice.  This is helpful when your signal is weak or

you're trying to bust a pileup.

14

Add Begin/End CR

RITTY can automatically add a CR when you go to transmit

mode and another when you return to receive.  This ensures that

your transmit text isn't printed next to noise characters.

(RITTY doesn't add a CR when you interrupt output with Esc or

Pause, nor when one's just been sent.)

AFSK Autotune

Select On to automatically track AFSK center frequency

to that of the received signal.  For narrow shift, the transmit

shift will be 170 Hz for received shifts less than 185 Hz;

otherwise it will be 200 Hz.  AFSK autotune is useful when

calling stations.  Turn it off when calling CQ so your transmit

signal stays put.  AFSK autotune works only when receive

autotune is enabled.

Freq Shift

This AFSK parameter is provided for special purposes.

Normally you won't need to set it because transmit-frequency

shift tracks changes you make to receive shift with the up- and

down-arrow keys.  (Transmit shift doesn't track fine changes

made with the left- and right-arrow keys.  This lets you fine-

tune a signal and still reply at the correct shift.)  If you

manually set transmit shift, don't forget that the value will

change whenever you alter the receive shift with the up- or

down-arrow keys.

While narrow shift today is standard for amateur RTTY,

wide shift plus ATC provides better immunity to signal fading.

See WIDE.DOC for details.

Stop Bits

You can select 1, 1.4, 1.5, or 2 AFSK stop bits.  1.4

really is 31/22 stop bits (31 ms at 45 baud), the standard for

Teletype machines at 45 and 57 baud.  Most amateur RTTY signals

still use this length today.  1.5 stop bits is common at the

other baud rates.  Some TNCs transmit 2 stop bits.  This slows

transmission and widens the receive window during which noise

can generate a false start pulse.  Nevertheless, RITTY provides

this length for testing or other special purposes.  Finally, you

can select 1 stop bit.  This length sometimes is used at the

higher baud rates and can noticeably speed transmission at 45

baud.  However, some decoders won't reliably synchronize to

signals with stop bits this short.

Whenever you change baud rate, RITTY resets the transmit

stop-bit length to 1.4 at 45 and 57 baud and to 1.5 at 50, 75,

and 100 baud.  If you manually set the number of transmit stop

15

bits, don't forget that the setting may change when you alter

baud rate.

The COM-port UART determines FSK stop-bit length.  It is

always 1.5 (or very close).

New Line

RITTY provides four choices for the character sequence

emitted when you press Enter or when RTTY by WF1B sends CR.  CR

is the shortest possible sequence; use it during contests when

you're determined to minimize overhead.  The default CR LTRS

sequence refreshes the shift state at the beginning of each

line.  (RITTY sends FIGS instead of LTRS when the first

character of a new line requires it.)  Both of these sequences

work fine when decoded by terminal programs that won't overprint

lines.  CR LF LTRS works even for primitive terminal programs

that don't inhibit overprint.  Finally, use CR CR LF LTRS LTRS

when transmitting to a Teletype machine.  This sequence provides

time for the carriage to return to the first print column.

(However, it may cause some terminal programs to erroneously

double-space lines.)  RITTY limits the transmit line length to

72 characters when you select this sequence.

RITTY appends LFs to consecutive CRs when New Line is CR

or CR LTRS to enable blank lines to be reproduced by decoders

that properly ignore consecutive CRs.

Tones

This is the same parameter as in the Receive Options

menu.  Sharing the parameter prevents inadvertent reverse

transmission.

---- BAUDOT PUNCTUATION ----------------------------------------

Unfortunately, punctuation for the Baudot code isn't

completely standardized.  RITTY provides two punctuation sets.

The first set is used on U.S. Teletype Corporation printers.

The second is based on the International Telegraph Alphabet No.

2.  RITTY represents the unassigned FIGS-D, FIGS-F, FIGS-G, and

FIGS-H ITA2 codes by a centered dot (extended-ASCII 249).  These

codes may have country-specific meaning.  You'll find both

punctuation sets in use on the air today.  To change sets, press

P in receive mode and then up- or down-arrow.  The selected

punctuation set also is active on transmit.

You can modify individual punctuation marks in either

set.  Use the left- and right-arrow keys to select a punctuation

mark.  Then type your replacement.  To enter an extended-ASCII

character, press Alt while typing its three-digit code on the

numeric keypad.  The bell character is ^G.

16

If while transmitting you press a key not defined in the

current Baudot character set, RITTY won't echo it to the screen.

Instead, it emits a short beep to alert you that the character

can't be transmitted.

You can create a composite punctuation alphabet by

substituting ' for bell in the U.S. punctuation set.  This code

will print all signals on the air intelligibly.  (RITTY will

transmit FIGS-J for '.)

---- SCREEN COLORS ---------------------------------------------

Press C to set screen colors.  (To set them all, enable

Show Control Codes in the Receive Options menu.  Then enter some

transmit text and press Esc before it's all sent.)  Select a

screen item with the left- and right-arrow keys (an item blinks

once when selected).  Then use the Home, End, up-arrow, down-

arrow, PgUp, and PgDn keys to cycle through the 64 intensities

available for each of the red, green, and blue color components.

Press Esc to exit the Colors menu.

---- IMPLEMENTATION NOTES --------------------------------------

RITTY ignores the Baudot BLANK character on receive and

transmits it only as a substitute for non-Baudot characters in a

message or file.

RITTY never overprints a line; when it receives CR

without LF, it advances print to the next line.  When it

receives consecutive CRs, it doesn't erroneously multispace

lines.  RITTY displays all characters received; when it receives

more than 80 on a single line, it wraps print to the next line.

Whenever you enter more than 80 characters on a transmit

line, RITTY wraps the display to the next line but doesn't

transmit any control codes.  Exception:  When the new-line

sequence is CR CR LF LTRS LTRS, RITTY wraps text after 72

characters and transmits the new-line sequence.

RITTY emits three spaces when you press Tab.  This isn't

true tabulation but it does provide a convenient way to indent

paragraphs.

When you switch to transmit, RITTY first sends LTRS for

synchronization and then CR, if programmed.  Finally, it sends

LTRS or FIGS before the first text character.  When you return

to receive, RITTY adds only CR, if programmed.

Except when generating pure tones with Alt-M or Alt-S,

RITTY outputs LTRS diddle when it has nothing to send.  See

DIDDLE.DOC for the reason.

RITTY programs the sound card for approximately 6000

samples per second.  This allows a maximum input or output

17

frequency of 3000 Hz.  Creative Labs sound cards have built-in

antialias filters that do a pretty good job.  There is some

input roll-off above 2500 Hz, but with ATC this has no effect on

received signals, even for 850-Hz shift at a center frequency of

2210 Hz.

I measured the following output response for my Sound

Blaster 16:  -3 dB at 2650 Hz, -6 dB at 2710 Hz, and more than

12 dB down above 2800 Hz.  If you transmit 850-Hz shift at a

center frequency of 2210 Hz, the space signal will be 3 dB

weaker than the mark signal.  You may want to lower the center

frequency to equalize tone levels for this particular case.  For

narrower shifts your IF filter likely will cause more roll-off

than the sound-card antialias filters.

---- USE WITH RTTY BY WF1B -------------------------------------

RITTY can act as a modem for version 2.25 or later of

WF1B's RTTY contest-logging program.  To use it this way, first

start RITTY and check all menu settings.  Settings recommended

for contest use are listed below.  Exit RITTY and type RITTY I

to install the program in memory.  Then start WF1B.  Select the

K6STI TNC in the work sheet.  The tuning indicator is a

simplified version of RITTY's FFT display.  Tune until the

signal pips are within the marker bars.  After exiting WF1B,

type RITTY U to uninstall RITTY from memory.

You can automate this procedure with a little batch

file.  See CONTEST.BAT for an example.  (CONTEST.BAT assumes

that after checking all menu settings, you copied RITTY.INI to

CONTEST.INI.  CONTEST.BAT uses the FONT.EXE utility to provide

enhanced screen fonts for WF1B.  Type FONT for more

information.)  RITTY sets ERRORLEVEL to 1 when it encounters an

error (e.g., can't locate sound card).  You can test ERRORLEVEL

in a batch file to take evasive action.

If you can't load RITTY and RTTY by WF1B simultaneously,

try to make upper-memory blocks available.  With DOS 5 or later,

RITTY can use up to 23K of upper memory to free space for WF1B.

If you have lots of upper memory, try the LH RITTY I command to

load RITTY entirely there.  See your DOS manual for memory-

management information.

I recommend the following parameter settings for

contests.  Settings marked -- don't matter.

Receive Shift           182 Hz

Punctuation             Default top set with bell replaced by '

Colors                  --

18

Setup

Source                  As required

Text Font               --

Zero Style              --

Audible Bell            --

WF1B Det Key            ~ (if your keyboard has it)

Input Level             As required

Output Level            As required

Output Delay            As required

Output                  AFSK (if you can use your CW filter)

COM Port                As required

Aux Port                --

Port X Address          As required

Port Y Address          --

Port Y IRQ              --

Center Freq             Center of your CW-filter passband

RTTY Baud               45

Rated TX Duty Cycle     100%

Receive Options

Flywheel                Auto

ATC                     Auto

Detector                Narrow

Wide Det BPF            200 (Off if your CPU is too slow)

BPF                     On (Off if your CPU is too slow)

Autotune                -- (control with Alt-T & Alt-F in WF1B)

Autotune Width          500 Hz

Unshift on Space        Depends on contest exchange

Ignore Isolated LF      Yes

Display Both Cases      --

Show Control Codes      --

Tones                   Normal

Squelch                 Off

Waveform                --

Transmit Options

Word Output             No

Duplicate LTRS FIGS     No

Add Begin/End CR        Yes (unless you add them in WF1B)

AFSK Autotune           -- (control with Alt-T & Alt-F in WF1B)

Freq Shift              182 (set only after you set RX shift)

Stop Bits               1.4

New Line                CR LTRS

Tones                   Normal

Be sure to set RITTY's center frequency to the center of

your CW-filter passband.  Typical CW filters have poor group-

delay characteristics near their passband edges.  These filters

can seriously distort FSK signals even when their amplitude

response is flat.  Target the filter center to keep signals as

19

far from the passband edges as possible.  If you have a choice

between 250-Hz and 500-Hz filters, use the latter unless you've

carefully evaluated the time-domain response of the narrower

filter on FSK signals with the waveform display.

The recommended fixed transmit and receive shifts better

accomodate the 200-Hz shift used by unmodified multimode

controllers.  Using filters spaced 182 Hz does not compromise

reception of 170-Hz signals because it reduces interchannel

crosstalk.  182 Hz is within 0.2 Hz of the shift that causes 45-

baud mark/space crosstalk to vanish for matched filters.  If

you're a shift bigot and insist on using 170 Hz, you'll degrade

receive sensitivity about 2 dB for 200-Hz signals.  Degradation

is less than 1 dB at 182 Hz.  Better still, use autotune for

optimal tuning.

Press Alt-T in WF1B to cycle through autotune off, RX

autotune, and RX plus TX autotune (the last state doesn't appear

when using FSK).  When autotune is enabled, the tuning-indicator

scale varies with received shift so that tuning still is correct

when the signal pips line up with the markers.  Disable TX

autotune when calling CQ so your transmit frequency doesn't

wander.  To freeze and unfreeze autotune in WF1B, press Alt-F.

(Autotune locks to signals twice as rapidly in WF1B as it does

in standalone mode.)

The autotune window is 500 Hz wide by default.  You can

decrease its width so that QRM is less likely to capture the

filters (but you'll have to manually tune some callers).  If you

make the window something like 250 Hz, autotune acts more like

automatic fine tuning.

Press Alt-~ in WF1B to toggle between narrow and wide

detectors.  WF1B displays "Polar" when the wideband detector is

active.  (~ is intended to suggest a wavy signal).  It's a good

idea to start WF1B with the narrowband detector and its BPF

setting.  RITTY then will restore this setting whenever you

toggle back to narrow with Alt-~.  (If your keyboard doesn't

have the ~ key, select - or \ instead.)

Both the U.S. and ITA2 punctuation sets are adequate for

contest use.  The recommendation above makes operating more

enjoyable by silencing annoying bell codes generated by noise.

But here's an interesting idea from N1RCT:  Convert punctuation

not normally used during a contest to its corresponding LTRS

code.  If that code wasn't sent, it's noise anyway.  You must

decide which characters to convert; for example, you'll never

need an apostrophe for a contest exchange, but you might want

one while chatting with a buddy during a break in the action.

Many modems don't anticipate the use of unshift-on-space

on receive.  After sending a number and a space, these modems

don't resend FIGS before sending another number.  Therefore,

contest exchanges with more than one number (or numerical

repeats) may become garbled.  If the only number in the exchange

is RST, it's probably best to enable unshift-on-space.  (Just

20

remember that TOO means 599).  If the exchange involves

additional numbers, you may want to disable the feature.  You

can always use Alt-L in WF1B to force the shift state back to

LTRS (after the fact).

You can make your transmit signal more robust in noise

and QRM by having RITTY duplicate each LTRS and FIGS it sends.

However, this slows transmission.  The recommended setting

assumes your transmit signal is loud and clear.  Consider

duplication if you run low power during a contest.

RITTY does not echo to WF1B the CRs it adds to the

beginning and end of a message.  This conserves screen space.

Make certain you do not set the number of transmit stop

bits to 1 or 2 for a contest.  Some multimode controllers won't

reliably copy signals with 1 stop bit no matter how loud they

are.  Using 2 slows your transmission speed by 8% and makes your

signal more vulnerable to false start pulses caused by noise.

The only multiline messages you're likely to transmit

during a contest are long CQs.  Your callsign suffix resets the

shift state to LTRS just prior to each new line.  However, if

the LTRS is missed, everything up to your callsign suffix on the

next line will be interpreted as FIGS (assuming no unshift-on-

space).  Using CR LTRS for the new-line sequence is cheap

insurance.  Anything longer is overkill.

---- COPYRIGHT AND LICENSE -------------------------------------

This software is copyrighted.  It's licensed, not sold.

When you purchase a license for a software product, you're

granted the right to use the software under certain conditions;

ownership of the software remains with the copyright holder.

Licensing software is different than purchasing a tangible

object that you may use or dispose of as you wish.

This software is licensed to individuals at discount for

amateur use.  It has been provided to you on the condition that

you abide by the following:

1.  You will not use the software for professional, business,

government, military, or institutional purposes.

2.  You will not sell, rent, lend, give away, or otherwise

transfer your RITTY.LIC license file to others.

Brian Beezley, K6STI       (760) 599-4962

3532 Linda Vista Dr.        0700-1800 PT

San Marcos, CA 92069        k6sti@n2.net

21