PACTOR

Copyright 1997 by Brian Beezley, K6STI

All Rights Reserved

PACTOR.........................................1

Setting Up.....................................1

Operating......................................2

Monitor Mode...................................3

FEC Mode.......................................4

Linked Mode....................................5

Displays.......................................7

Receive Options Menu...........................7

Transmit Options Menu..........................8

---- PACTOR ----------------------------------------------------

PACTOR is a data-communications protocol developed by

Spezielle Communications Systeme GmbH in Germany and licensed

for use in RITTY.  This document describes PACTOR mode.  See

RITTY.DOC for general information about RITTY and details about

RTTY mode.

PACTOR uses two-tone FSK like RTTY, but offers higher

data rate, data compression, error correction, and the extended-

ASCII character set.  Instead of transmitting individual

asynchronous characters, PACTOR sends packets of information

using synchronous transmission.  The receiver acknowledges

reception of each packet before the next is sent.  Packets

contain a special tag that permits the receiver to determine

whether data has been correctly received.  The receiver can

request retransmission of bad packets.  PACTOR transmits at 100

or 200 baud and automatically changes speed according to channel

conditions.  It uses Huffman coding to compress text for faster

throughput.

---- SETTING UP ------------------------------------------------

The PACTOR protocol requires your transceiver and

amplifier to switch quickly between transmit and receive.  VOX

can't be used because it switches too slowly.  Instead, use a

COM-port output bit to key your push-to-talk (PTT) line.  See

RITTY.DOC for details.

You can use AFSK or FSK with PACTOR.  Unlike RTTY,

PACTOR FSK requires an auxiliary COM port to provide timing.

This port cannot be the one used for data output.  However,

since data isn't sent to the auxiliary port, you can use the COM

port of an existing device (for example, a modem or serial

mouse).  Select COM ports in the Setup menu.  RITTY assumes that

auxiliary COM ports 1-4 use standard IRQs.  Select port Y to

specify a nonstandard port address or IRQ.

PACTOR works best with a frequency shift of 200 Hz.

RITTY always emits narrow-shift AFSK at this shift.  If you use

FSK and your transceiver generates 170-Hz shift, signal-to-noise

ratio (S/N) will degrade 1.6 dB at 200 baud and 1.4 dB at 100

baud for a matched-filter demodulator.  Use FSK only when you

can't use your CW filter in LSB mode.  RITTY provides a sharp

input bandpass filter for use instead of a CW filter.

Set Output Delay in the Setup menu to the time it takes

your transceiver and amplifier to switch from receive to

transmit.  The best way to determine this number is to examine

RF output with an oscilloscope while applying a steady input

tone and repeatedly switching from receive to transmit.  If you

can't make the measurement, the 25-ms default should work with

most equipment.  Setting this parameter unnecessarily high can

inhibit long-distance PACTOR links.

1

Set Rated TX Duty Cycle in the Setup menu to your

transmitter's rated full-power output percentage for extended

transmissions.  When using AFSK this parameter allows RITTY to

transmit brief control signals at full power while reducing

output for packets and FEC to avoid overheating your

transmitter.  For example, if your 100-watt transceiver is rated

at 50 watts output for extended transmissions, enter 50%.  RITTY

then will output AFSK at the following relative power levels:

Control Signals   100 watts

LP Packets         73

SP Packets         65

FEC & RTTY         50

---- OPERATING -------------------------------------------------

Use Alt-/ to switch between RTTY and PACTOR modes.

Press S for the Setup menu, R for the Receive Options menu, and

T for the Transmit Options menu.

PACTOR has three operating modes:  Monitor mode lets you

tune around and monitor QSOs and CQs.  FEC mode lets you call CQ

or broadcast a message in much the same way as RTTY.  Finally,

linked mode is for QSOs.  When linked you communicate with one

station and the PACTOR protocol provides error correction,

automatic speed change, data compression, etc.  When you end a

link or finish a FEC transmission, RITTY returns to monitor

mode.

PACTOR links require both callsigns.  Your callsign is

provided by your RITTY.LIC license file.  To establish a link to

another station, press L and enter his callsign.  RITTY

immediately begins to transmit a connect request.  RITTY listens

between the request packets for a response.  When "Linking" on

the bottom screen line changes to "Transmit," you're linked with

the other station and you can begin to type a message.  RITTY

transmits your message in packets about one second long.  Each

packet is acknowledged by a brief control signal (CS) from the

far end.  Your transceiver switches to receive to listen for the

CS and then switches back to transmit.

When you're ready for the other station to respond to

what you've typed, press the Pause key to turn the link around.

You'll see "Transmit" change to "Receive" and you'll begin to

display his response.  Normally the other station will turn the

link back to you when he's done typing, but you can interrupt

and grab the link by pressing Pause.

To terminate a link, press ^Q (Ctrl-Q) while

transmitting.  This sends a special QRT packet to the far end,

which is then acknowledged.  If signals are weak and you can't

receive the acknowledgment, press ^Q again to return to monitor

mode.  If you press ^Q while receiving, RITTY returns to monitor

mode without notifying the far end.

2

To call CQ, press Pause in monitor mode to enter FEC

mode.  Whatever you type is transmitted.  RITTY repeats each FEC

packet to facilitate reception.  (Duplicate text isn't displayed

by someone monitoring your transmission.  You can set the number

of repeats in the Transmit Options menu.)  Press Esc or Pause to

return to monitor mode and await a connect request.  When

someone connects to you, his callsign appears in gray in the

lower-left screen corner.  The station connecting to you

transmits first.

As in RTTY, press F1 to F12 in monitor mode to prepare a

canned message.  Press the function key while transmitting to

send the message.  Press Alt-F1 to Alt-F12 while transmitting to

edit a message.  You can access the Transmit Options and Receive

Options menus while transmitting by pressing Alt-T and Alt-R.

You cannot access the Setup menu while transmitting.

---- MONITOR MODE ----------------------------------------------

RITTY uses concurrent 100- and 200-baud bitstream

decoders, each with optimal SIN(x)/x channel filters matched to

the baud rate.  When a monitored signal changes speed, RITTY

automatically displays the output of the proper decoder.

The callsign of the station someone is trying to link

with will appear in the lower-left screen corner in dark blue.

The callsign is transmitted only during connect attempts, not

during QSOs in progress.

Three red dots in text indicate that at least one packet

could not be decoded and text is missing.  This is common during

signal fades in monitor mode.

It's normal to see a few garbage packets of nonsense

text each hour when monitoring noise.  This occurs when the

special CRC (cyclic redundancy check) packet tag erroneously

indicates that decoded noise constitutes a valid packet.

Once RITTY receives two consecutive CRCs, it locks

(synchronizes) to the bitstream and preferentially decodes

synchronized packets.  It remains locked until it receives

several consecutive bad packets (Unlock Count in the Receive

Options menu).  A small count lets RITTY quickly relock to a new

bitstream but prevents it from hanging in as long for marginal

signals.  RITTY immediately unlocks when it detects link

turnover or QRT.  RITTY erases the baud rate annotation when it

unlocks.

As described below for linked mode, when locked to a

bitstream RITTY can repair packets.  It displays repaired text

in dark blue.  Because many PACTOR systems transmit FEC packets

with irregular timing, RITTY does not attempt to lock to FEC

bitstreams and cannot repair FEC packets.

3

To extract maximum information when monitoring a weak

signal, enable Monitor Bad Pkts in the Receive Options menu.

When locked to a bitstream RITTY then will decode all packets

regardless of CRC.  It displays text from bad packets in red.

There's often useful information within the junk.  Monitoring

bad packets automatically enables packet repair.

"Good" means a packet was received without error or was

repaired.  "Bad" means at least one error remained.  These

annotations use the same color scheme as the text.

When QRM is heavy, use the Receive Options menu to

engage the input bandpass filter.  This 200-tap, linear-phase,

FIR filter is optimized for the 200-baud PACTOR signal spectrum.

The filter is 500 Hz wide at -6 dB, 800 Hz wide at -60 dB, and

has an ultimate rejection of more than 100 dB.  This filter

should work better than a typical CW filter (the bandwidth is

about the same, the shape factor and ultimate rejection probably

are better, and the group delay is much better).  The BPF

slightly degrades receive sensitivity, but the effect is so

minor you probably won't notice anything amiss if you simply

leave the filter engaged all the time.  The filter requires

considerable computation and may not run on slower computers.

Tuning a PACTOR signal is less critical than with RTTY

because the channel filters are wider.

PACTOR interchanges the mark and space channels each

packet.  This allows use of LSB or USB without concern for tone

reversal.

Some stations use AMTOR FEC to call CQ rather than

PACTOR FEC, even though they expect a PACTOR QSO.  You won't be

able to copy these CQs.

---- FEC MODE --------------------------------------------------

FEC stands for forward error correction.  PACTOR

actually performs only error detection in this mode.  The

receive gap between transmit packets is eliminated and each

packet is repeated before the next is sent.  (Most stations send

each packet twice.)  Repetition gives the receiver a chance to

recover text when an error occurs.

While the most common use for FEC mode is to call CQ,

it's also handy when you have problems linking.  You can use FEC

to obtain a callsign or to QSY to another frequency.  It's also

sometimes used to make a final remark after link termination.

When conditions are poor, manually repeat your message (or

increase FEC repeats) to make sure it gets through.

As with RTTY, you can embed ^R in a message to end

transmission and return to monitor mode.  If you enter ^R while

typing ahead, RITTY will switch to monitor mode when it

encounters the character.

4

In both FEC and linked modes RITTY avoids Huffman

encoding whenever there is no throughput penalty.  This permits

more text to be recovered from bad packets.

---- LINKED MODE -----------------------------------------------

When linked, PACTOR provides a brief interval during

which the receive station replies with a control signal.

However, this window is too short to permit the CS to be

received by long-path propagation.  PACTOR provides an alternate

timing scheme with a wider window for use in long-path QSOs.  To

initiate a link with LP timing, press ! when entering the

callsign.  LP timing also is useful for short-path QSOs with

stations that use long transmit delays.

When sending a connect request, RITTY searches the

entire receive window for a control signal from the far end.

This provides a great opportunity for noise to masquerade as a

valid response.  When this happens RITTY switches from linking

to transmit.  If it doesn't receive another CS within three

frames, it reverts to sending a connect request.

When it receives a second CS, RITTY stays linked.  It

can then tolerate errors in the CS1 and CS2 control signals that

acknowledge packets.  If it can't recognize a control signal,

RITTY must repeat the last packet.  Allowing one or more CS bits

to be incorrect speeds data in noise and QRM.  You can specify

the number of hits allowed in CS1 and CS2 with the Transmit

Options menu.  RITTY allows no hits in CS3 (turnover) or CS4

(speed change) because false recognition of these control

signals may cause link failure.

Tips:  Press Esc to terminate a link attempt.  Press L

then ! to toggle LP timing without reentering the callsign.

Embed ^R in a message buffer to turn the link around or ^Q to

QRT the link.  Use the backspace key to correct typing errors

even after they've been received and displayed at the far end.

If you alter the input-filter setting while linked, the

signal-delay change will kill the link.  This also will occur if

you engage your CW filter, although you may be able to get away

with it at 100 baud.  Set your filters before you link.

The number displayed in the lower-right screen corner is

the delay between the end of your transmit packet and the

beginning of the received control signal.  This figure includes

equipment delay at both ends and propagation delay.

The number on the bottom line just to the left of center

is the difference between the far-end protocol clock and your

clock in parts per million.  RITTY displays this number only

when someone connects to you, not when you initiate a link.  The

measurement takes a while to stabilize.  If you notice a

consistent bias for many stations, modify the Clock Offset value

in the Transmit Options menu in the opposite direction.

5

RITTY implements memory-ARQ.  This error-correction

technique works as follows:  When a packet and its

retransmission both have errors, RITTY combines the packets by

summing the demodulated waveforms (quantized to 21- or 22-bit

resolution).  Combining packets increases S/N because the

signals add coherently but the noise adds incoherently.  (S/N

increases 3 dB when the packets have the same signal and noise

powers.)  If the combined packet still isn't error free, RITTY

saves it and requests another retransmission.  RITTY keeps

adding repeated packets to the combined signal until a received

packet is good or the combination is good (or either can be

repaired, as described next).  For each combination RITTY

calculates a summing coefficient that maximizes combined S/N.

RITTY can correct single-bit errors without requesting

retransmission.  It does this by flipping each bit of the

message in turn and retesting the CRC.  If the CRC becomes

valid, it accepts the message.  Packet repair speeds links

during difficult conditions.  RITTY also applies packet repair

to the memory-ARQ signal.  This provides six chances to recover

a packet repeated once.  Packet repair does increase the risk of

decoding a packet with undetected errors, but this seldom occurs

while linked.  You can disable packet repair from the Receive

Options menu.

Packet repair uses another trick to correct multiple-bit

errors under certain circumstances.  Packets for hand-typed text

usually contain fill characters to pad the packet to the proper

length.  Bit errors in the fill characters cause CRC failure

even though the characters aren't displayed.  When packet repair

is enabled and RITTY encounters a fill character, it overwrites

the remainder of the text with fills and then attempts to

correct single-bit errors.  This strategy permits text recovery

even when the padded part of a packet contains many errors.

Although RITTY does not perform fill-replacement for Huffman-

encoded packets, most TNCs generate ASCII packets for hand-typed

text.

"Good" means a packet was received without error or was

corrected.  "Bad" means the packet could not be recovered and a

retransmission was requested.  RITTY can display bad packets

(red text) in monitor mode but does not attempt to do so while

linked.

RITTY colors error-corrected text like this:  Dark blue

is packet repair, tan is memory-ARQ, and light blue is memory-

ARQ with packet repair.  RITTY colors "Good" the same way.  It

colors "Bad" purple when it repairs a packet but certain control

bits contradict other information.  (Although it obtained a

valid CRC, RITTY asks for a repeat since there's evidence it

incorrectly reconstructed important control data.)  During

transmit RITTY colors the CS annotation dark blue when it

recognizes CS1 or CS2 with errors.

RITTY can send and receive ANSI graphics but does not

display it in color.

6

Unlike ordinary TNCs, RITTY's matched filters provide a

3-dB S/N advantage at 100 baud compared with 200 baud.  RITTY's

speed-setting algorithm takes this gain (equivalent to doubling

transmit power) into account.  RITTY switches to 200 baud only

after receiving several completely filled packets without error

(Speed Count in the Receive Options menu).  Links always begin

at 100 baud.

---- DISPLAYS --------------------------------------------------

The left-hand display is a spectral tuning indicator

just like that for RTTY.  PACTOR receive-filter frequencies do

not autotune.  Use the arrow keys to alter them.

During receive the waveform display on the right shows

either the packet header (alternating bits) or the data samples

and decision threshold (ATC trace) for the entire packet.  The

header display provides a high-resolution snapshot of the

demodulated waveform.  It can reveal selective-fading

distortion, transmit start-up anomalies, and suppression of

leading bits by your noise blanker.  The packet display shows

the signal envelope and ATC action.  It can show distortion due

to fading, QRM, static crashes, and receiver AGC artifacts.

During transmit the waveform display shows the received control

signal.

---- RECEIVE OPTIONS MENU --------------------------------------

Timeout Count

This parameter is the maximum number of times RITTY will

send a packet without receiving acknowledgment.  It also is the

maximum number of consecutive bad packets during receive.

Finally, it is the maximum number of connect attempts.  When a

timeout count is exceeded, RITTY returns to monitor mode.

Speed Count

This parameter is the number of consecutive, completely

filled packets RITTY must receive before switching to 200 baud.

It also is the number of consecutive bad packets required to

downspeed to 100 baud.  (When packets aren't completely filled,

RITTY downspeeds more readily.)

Unlock Count

This parameter applies only to monitor mode.  It is the

number of consecutive bad packets RITTY must receive before

unlocking from one bitstream and becoming more receptive to

another.  When monitoring bad packets, RITTY can't recognize

another bitstream at all until it unlocks.

7

Callsign Hits

RITTY can tolerate errors in the received callsign (your

callsign) during a connect attempt.  This permits much faster

connects.  This parameter is the number of errors allowed in the

72-bit callsign comparison.  The default is 1.  Use 0 to connect

only when your callsign has been perfectly received.

Repair Packets

Select Yes to enable RITTY to correct single-bit errors

in received packets in both monitor and linked modes.

Monitor Bad Pkts

Select Yes to display text in monitor mode for packets

with invalid CRC.  This also inhibits relocking to a new

bitstream until several consecutive bad packets have been

received (Unlock Count), or the link turns over or QRTs.

BPF

The bandpass filter is optimized for 200-baud PACTOR

signals.  You can select it independently of the RTTY BPF.  The

filter lowers receive sensitivity by a small amount.

Waveform

Select Header to view the packet header (alternating

bits).  Select Packet to show the data samples and decision

threshold for the whole packet.

---- TRANSMIT OPTIONS MENU -------------------------------------

Timeout Count

This parameter duplicates that in the Receive Options

menu.  The two counts are always the same.

CS1/CS2 Hits

This is the number of incorrect bits allowed in CS1 and

CS2 while linked.  The default is 1.  To recognize CS1 and CS2

only when perfectly received, set the number to 0.  Control

signals are 12 bits long and differ in 8 bit positions.

8

FEC Repeats

This is the number of times RITTY transmits a packet in

FEC mode.  (If you want to repeat it once, enter 2.)  Nearly all

stations transmit FEC packets twice.

FEC Baud Rate

Nearly all stations transmit FEC at 100 baud.

Clock Offset

This parameter compensates for sample-rate variation

among sound cards.  Change its value when you notice a

consistent bias in the displayed clock offset when stations

connect to you (mild offset) or when links always hang (severe

offset).  To measure your clock offset, enter a value of 0,

output a 1000-Hz RTTY AFSK tone, and measure its frequency with

a high-resolution frequency counter.  A reading of 1000.145 Hz,

for example, means your clock is 145 ppm fast.  Enter an offset

of -145 ppm to compensate.

9