---------------------------------------------------------------------------

FlexNet documentation           TFEMU.EXE         (C)1996 Tom Sailer HB9JNX

Date: 4.2.96

Ident: TFEMU

TheFirmware emulator for PC/FlexNet

This TF-Emulator allows existing terminal-programs, written for the WA8DED/

TheFirmware command set to be used with PC/FlexNet. These programs include

GP, SP, TheBox, DX-Cluster and others.

TFEMU may be loaded 4 times maximum. Since PC/FlexNet only accepts one

program with a given name, TFEMU must first be duplicated, for example with

"COPY TFEMU.EXE TFEMU2.EXE". The different TFEMU recognize their mutual

existence and automatically dispatch incoming connects among them, so the

different TFEMU may run under the same mycall&SSID. The streams of the first

loaded TFEMU are filled with incoming connects first, up to the maximum

specified with ESC Y, then those of the second TFEMU and so on. If all

streams with a given mycall in all TFEMU's are filled, then a CONNECT REQUEST

message is generated. It is however unpredictable on which TFEMU this message

is output.

Parameter:

/?,/h    description of the parameters

/i       interrupt nummer (default 0xfd)

/dr      use DRSI interrupt format (TNCTSR-compatible (hopefully)) instead of

TFPC(N5NX) format and output port numbers in DRSI format (at link

state messages and monitor headers)

/dx      use DRSI interrupt format, and output port numbers in TFPCX format

/dm      use TFPC(N5NX) interrupt format, and output port numbers in TFPCX

format. If none of the /d parameters is used, then a TFPC(N5NX)

interrupt is installed and no port numbers are output

/b       buffer memory (min. 4kBytes, max. 48kBytes) in bytes

/p       Convert DRSI port numbers to FLEXMINI format. Works only with

FLEXMINI.

/l       lock out a call+SSID. This should help the sysops of a DX-Cluster

which is directly connected to a RMNC. The RMNC likes to poll the

DX-Cluster, which forces the Cluster sysop to put the digi callsign

into the lockout list. This may be circumvented if the /l option

is used. The cluster software then does not notice the RMNC polls.

(the frames are displayed in the monitor, but no connect request

link state message is generated)

The driver may be unloaded either with FLEX /U (which unloads also the other

PC/FlexNet components) or with KILLAPPL. The FlexNet channel parameters may

either be set with ESC T and ESC @MODE or with the FlexNet utility FSET.EXE.

Implemented TNC commands

ESC A [0|1]     Determines whether an LF is output after each CR (1) or not (0).

Affects only the received data. Default: 1

ESC C [Pfad]  * Connect a station. A connect command on channel 0 (monitor/UI)

sets the path for UI packets. DRSI port numbers (for

example 0:HB9W) are removed, since FlexNet knows where to go...

ESC D         * Disconnects a QSO.

ESC E [0|1]     Determines whether each input is copied to the output. This

affects only the terminal mode. Default: 1

ESC F         * Outputs the FRACK (round trip time) value, if connected.

Otherwise it outputs some fantasy value for compatibility

reasons

ESC G [0|1]     Only in host mode. Polls the specified channel. If the

channel 255 is polled, a list of channels with pending data

or link state messages is returned (Extended host mode polling)

ESC I [Call]  * Sets the source callsign (mycall)

ESC JHOST[0|1]  Switches the host mode on and off. The host mode is used by

most packet radio programs

ESC K[0|1|2]    Switches the time stamps on and off. ESC K0 switches all time

stamps off, ESC K1 switches the time stamps on for link state

messages, and ESC K2 additionally time stamps monitor headers.

ESC K HH:MM:SS  Sets the TFEMU time. The accuracy of the TFEMU time depends on

the accuracy of the FlexNet kernel clock.

ESC K TT.MM.JJ  Sets the date (european format)

ESC K MM/DD/YY  Sets the date (american format)

ESC L[n]      * Displays the channel state. This command behaves differently in

host mode.

ESC M[UISCN+-.] Switches the monitoring on/off. U monitors UI packets, I

monitors information packets (connected) and S monitors super-

visory packets (such as RR, RNR, REJ and so on). C enables

monitoring while a connection is pending. N switches the

monitoring off. C has no effect in host mode.

In terminal mode, only packets with PID F0 are displayed.

With + and - channels may be specified. - suppresses the

following channels, and + monitors the specified channels

exclusively. ESC UISC-10 for example monitors all packets on

all channels except for channel 10.

ESC N           Maximum number of retries. Returns always 10 (for compatibility)

ESC O         * Returns the MAXFRAME value of the stream (if connected)

ESC P [port]    Returns the p-persistence value (256/number of users)

ESC QRES        Resets the software. The default parameters are set

ESC S[n]      * Displays or sets the active stream (terminal mode only)

ESC T [ch:]txd  Sets the TX delay on flexnet channel ch (0..15) to txd*10ms

ESC V           Displays a string with the program version

ESC U num text  Sets the unattended mode and the text to be sent.

If num = 1, then the text is sent to all incoming connections

ESC Y num       maximum number of incoming connects

ESC @B          Returns the amount of free memory

ESC @CAL ch min Enables calibration mode on channel ch for min minutes.

min=0 disables calibration prematurely. In calibration mode,

In calibration mode, zeroes and ones are sent alternatingly.

ESC @L1ST [po]  Returns the L1 statistics of the specified port

PO=port,ID=driver identification,TXF=TxFrames,RxF=RxFrames,

TE=TxErrors,RE=RxErrors,RBE=RxBufferOverruns,IOE=IOErrors

ESC @L2ST [po]  Returns the L2 statistics of the specified port

PO=port,TD=TxDelay,QSO=number of QSOs,USR=number of stations on

the QRG,TIFR=transmitted info frames,RIFR=received info

frames,TKBY=transmitted kilobytes,RKBY=received kilobytes,

QTY=quality (in %)

The values correspond to those in the FlexNet infobox

statistics

ESC @MODE ...   Sets the channel mode. The format is compatible to the infobox

mode command.

ESC @S[n]     * Displays the stream connection state

ESC @U[0|1]     Specifies whether the poll flag should be set in UI packets

ESC @WDOG[n]    Displays or sets the watchdog. 0 turns the watchdog off.

5..60 sets it to the specified number of minutes. The

application is then required to send at least one command,

data or a line to TFEMU, otherwise the PC will boot.

This feature could be useful for unattended stations, such

as BBS, Cluster and so on.

* This parameter may be set individually on each stream

Now incoming connections are taken also if TFEMU is in background (i.e. no

Terminal program is running). This works if there is enough free memory

and the foreground application behaves cooperatively enough.

(not exhaustively tested)

Differences to the Original-Firmware

====================================

(as far as they are known...)

- Frame reject link state messages are not followed by the three reason

bytes, because PCF does not indicate them

- The commands F (Frack) N (max. number of retries) O (MAXFRAME)

P (PPersistence)do not set these parameters. For the monitor channel, they

return some random number

- Some less important firmware commands are not implemented

- Monitor: AX25 packets with an invalid control byte are displayed as ??

instead of ?xxH, where xx would be the hex representation of the control byte

Interrupt formats

=================

TFPC(N5NX) interrupt

--------------------

The TFPC interrupt may be recognized at the string "N5NX" three bytes after the

address where the vector points to. The string is not terminated with a zero

byte!

Following functions are available:

AH=1  Return if a character is available

AX=0: No character available

AX=1: Character available

AH=2  Read a character. The character is returned in AL. Should only be called

if AH=1 returned AX=1!

AH=3  Write character in AL to TFEMU

DRSI interrupt

--------------

The DRSI interrupt may be recognized with the following bytes:

0x53 0x1e 0xbb 0x?? 0x?? 0x8e 0xdb 0x84 0xe4 0x74 0x20

Following functions are available:

AH=0  AH=0: No Character available

AH=1: Character available, which is returned in AL

AH=1  Write character in AL to TFEMU

Returns AH=0

Special routines

----------------

These routines are specific to TFPCX, but nevertheless they are needed by

some application programmes (see TERM below) These routines are available

at both interrupts

AH=0xfb  Returns: AL=Highest active port number+1

AH=Number of streams

AH=0xfc  Returns: AL=Bit mask of the ports with active DCD

(p.Ex. if(_AL&1) printf("Channel 0 has an active DCD"); )

AH=Bit mask of the ports with active PTT

AH=0xfd  Returns: AX=0 TFEMU is busy, nothing should be sent

AX=1 TFEMU is not busy

AX=0xfe  Rueckgabe: AH=2    Major version number (TFEMU emulates TFPCX v2.10)

AL=0x0a Minor version number

The following should be checked

- Is the format of the link state messages and the monitor headers really

correct

- Do other programs also work, except the ones mentioned below, especially

DRSI programs

- Is the output of the commands correct (except for some common sense

restrictions)

Success list (at least partial, hi)

============

TFPCT.COM v2.10 written by OE1HHC and OE3GMW

---------------

This is a simple terminal program, written by the TFKISS authors, which

uses the TFPC(N5NX) Interrupt. No anomalies known.

Reports by HB9JNX@HB9W

GP.EXE (Graphic Packet) v1.61 written by DH1DAE

-----------------------------

This is a host mode terminal program with graphical user interface (DOS) with

many features. It works with an without the -d options. Needs a lot of DOS

memory so EMM386 is most likely necessary. Uses the extended host mode. No

anomalies known.

Reports by HB9JNX@HB9W, DK7WJ@DB0GV, DG9FDL@DB0SRS, DL3FDU@DB0AIS

THS.EXE v2.50 written by HB9CVV

-------------

This is a menu driven DOS DRSI hostmode program. It exists quite a long

time. It works with the -dr option. No anomalies known.

Reports by HB9JNX@HB9W

TERM.EXE v10.36 written by DL5FBD, Gerd-Michael Ritter

---------------

Simple DOS terminal program, which is written in Turbo Pascal and whose source

code is available (including a serial interface library that supports "standard"

serial interfaces and the TFPC(N5NX) interrupt). Works now since TFEMU supports

the special function 0xfd (transmitter handshake). TERM uses this function

which is otherwise only supported by TFPCX without testing if this function

really exists. (TERM would have to ask for the TFPCX version and if the

returned value makes sense (it is neither 0, 0xffff, nor 0xfdxx), it may

use these functions)

Reports by HB9JNX@HB9W, thanks to DG9FDL@DB0SRS, Michael for his hint

SP.EXE Demoversion 9.00 written by Sigi Kluger DL1MEN (commercial)

-----------------------

Works as far as I may judge it. SP uses the ESC @B command for the transmitter

flow control, therefore TFEMU must(!!) have enough buffer memory, at least

16kByte, -b0x4000!

This should solve the problem with sending files

Reports by HB9JNX@HB9W, thanks to DG9FDL@DB0SRS, Michael for his hint

TOP.EXE v1.47 written by DF8MT

-------------

A superficial test did not show any problems...

Reports by HB9JNX@HB9W, thanks to DG9FDL@DB0SRS, Michael for his hint

THEBOX v1.9a written by DF3AV et al

------

TheBox may be started. WARNING: THEBOX DOES NOT TEST IF A TF EMULATOR

IS LOADED, AND CRASHES IF NOT...

TFEMU must be started with -i0xfe! Furthermore, TheBox uses the ESC @B command

for the transmitter flow control. Therefore, TFEMU must have enough buffer

memory, at least 16kByte (-b0x4000)! Otherwise, it seems to work (as far as

I can judge it...)

Reports by HB9JNX@HB9W

AA4RE BBS v2.9 written by AA4RE

---------

This mailbox also doesn't test if a WA8DED emulator is loaded! (This seems

to be a mailbox "feature"). Otherwise it works, just configure it as described

for DRSI's PC*PA adapter, start TFEMU with -dr -i0xff (the com number then

also has to be 255!)

Reports by HB9JNX@HB9W

PacketCluster v5.4-4 written by Pavillion Software (commercial)

-------------

Install as described for DRSI's PC*PA-Adapter. First tests were immediately

successful, more thorough tests will follow. cf /L option.

Example: the DX-Cluster HB9W-8 is connected to the RMNC HB9W 9-10. TFEMU

should be started with: TFEMU -DR -I=0xff -L=HB9W-9

Reports by HB9JNX@HB9W

FBB 5.15c written by F6FBB

---------

load-high FLEXNET.EXE, KISS.EXE, and TFEMU.EXE in your autoexec.bat. TFEMU

doesn't need any parameters, except a larger buffer size if you have an

overloaded BBS.

Concerning FBB, simply validate interface 5 on PORT.SYS (interface previously)

used with TFPCX/TFPCR. All INITTNCx.SYS, MAINTx.SYS, and BEACONx.SYS should

then use TFEMU commands (explained in this DOC file) : don't mention

TxDelay, Persist, etc... :-)

All tests have been done with FBB 5.15c linked to a FlexNet digipeater

on a 9k6 full duplex kiss wire link.

To know more, contact F5PJE @ F6BIG or F1UNA @ F5KPO.

Report courtesy by Eric F5PJE

Unsuccessful programs

=====================

TNCTERM.EXE (supplied with the DRSI driver)

-----------

This is a very simple terminal program. It does not work. It does not

recognize the TFEMU interrupt. Does not work also with other WA8DED emulators,

such as G8BPQ DED Emulator

Reports by HB9JNX@HB9W

Autor:  Tom Sailer

Weinbergstrasse 76

CH-8408 Winterthur

Packet: HB9JNX @ HB9W.CHE.EU

EMail:  sailer@ips.id.ethz.ch