7Plus Sender for Tpk v_2.0

^^^^^^^^^^^^^^^^^^^^^^^^^^

NOTICE: If you have already installed  7PSTPK v_1.0,  please remove its

configuration file before installing this new version.

Aim of the programme

^^^^^^^^^^^^^^^^^^^^

This programme is intended to help  the transfer of splitted files

(of 7PLUS programme),  by means of a proper  split capability, and

then, automalically, to perform the importation  of splitted files

into the list of messages to be forwarded by TPK programme.

After now and then,  it is  unnecessary  to  split  a  file  (or a

programme) into a specified number of files  of a given size,  and

then  send  them  to  your local BBS  by using  usual  procedures,

because now, TPK programme is able to perform this task, using  an

automated capability on forwarding splitted-message files.

Files of this archive

^^^^^^^^^^^^^^^^^^^^^

7PSEND.EXE      The executable file

7PSEND_E.DOC    This file

During the programme execution, temporary files  are automatically

generated by the programme itself; these files may  have different

denomination such as: .P01, .P02  etc. for the split files;  files

TEMPOR, SWAPP.7PS, .7PL and .INF,  are created by the programme if

the space on disk is extremely little.

In that case, there are high probabilities  for a blocking (crash)

of 7PSEND programme.

If this is what happens,  don't  worry at all,  because no problem

arises, and nothing will be destroyed.

Interaction between 7PSEND and TPK

^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^

7PSEND (by means of 7PLUS aide)  provides the splitting of a given

file,  up-to-dates  MSG_PRIV.LST  and  MSG_PRIV.NUM  files  of TPK

programme, creates and/or up-to-dates a TXMSG.TPK file, and at the

end, generates a message file into a format  that closely  matches

TPK   programme  and  closely  correlated  to  the  aforementioned

split-files.

Where do arrange programme-file?

^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^

This programme may  be arranged everywhere, in fact  configuration

file indicates to 7PSEND where to find the file allocation.

In any case,  I suggest to create a dedicate  subdirectory on that

of TPK programme  (I for example had created a subdir  of  \TPK181

denominated 7PS). The reason why  I give this instruction  depends

upon a change of denomination arised  some times ago, because many

users of 7PLUS renamed that programme  7P.EXE to allow to generate

a file 7PLUS.BAT containing appropriate switches to be released at

7PLUS.EXE to permit the deletion of files after their rebuilding.

Again,  in any case,  to  perform a  good job (a clean job) and to

have no risks, I suggest to create  a dedicated  subdirectory  and

put into it,  "ALSO"  7PLUS.EXE programme  which  you  can find in

every BBS.

For my tests, I  used  7PLUS v2.02  and  7PLUS v2.10, gaining very

good results in both cases.

However   this  programme  works  fine  "ONLY"  with  versions  of

7PLUS.EXE subsequent the version 2.0.

How do correctly operate this programme?

^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^

This programme,  in the most cases,  cannot  be  run  "under"  TPK

operation  (insufficient memory),  and then it must be run  before

starting TPK programme.

7PSEND is very easy to understand,  in fact you  must reply to the

proposed questions and giving, at  the end,  confirmation  to your

answers.

Even though it has been  conceived  to  work  by means of  "mouse"

facility,  7PSEND  works fine also without it,  in  fact  you  can

navigate the floating point by pushing  the  "TAB"  key  and  then

proceeding  to  mark  the  choosen  options  by  pushing  the  key

combination "ALT+<marked letter>".

Configuration menu

^^^^^^^^^^^^^^^^^^

Choose the parameter to be filled-in/changed, then, using  UP/DOWN

arrows keys or by clicking  the mouse on the  sign  ( ),  and when

this becomes  (*),  hit the combination keys  "ALT+M"  or click on

[SAVE].

Fill-in your CALL-SIGN (i.e. IW0DOP)

Fill-in the known BBS  (i.e. I0VNR-8)

Fill-in the PATH of  MSG_PRIV.LST  and  MSG_PRIV.NUM  files  (i.e.

C:\TPK181\LISTS).

Note that the aforementioned files are generated by  TPK programme

when  BROADCAST option of FBB is released, so you  MUST  have this

function validate, if not, you cannot use 7PSEND too!

Fill-in maximum AMOUNT of bytes for each split-file (this value is

very  much  important  because  indicates  to  7PSEND  the maximum

dimension   for  "each  segment"   (i.e.  the  maximum  split-file

dimension).

I'll suggest to you to follow this simple rule: if the transfer of

files concerned is addressed to your local area,  you may use high

values  for  each  split-file,  if instead your transfer  concerns

national or even international areas, values MUST  be necessarily,

quite low (i.e. 2000-3000 bytes for each split-file).

Fill-in the PATH to access to files, in the eventuality that

correction requests arise from remote stations (i.e.

C:\TPK181\CAPTURE).

After that you must save  these parameters  by clicking on  [SAVE]

and then [END].

At this point the programme  generates a configuration  file named

7PS_TPK.CFG into which are stored the above informations.

On my  experience,  in a  national  transfer  you must exect  some

correction requests, so having in  the past,  experienced  such as

corrections made  "by hand", I can tell you what big an effort and

meticolousness are necessary to perform this hard task.

Today thanks to  7PSEND  capabilities  this task  is automatically

done  so,  once  transfer  is got,  you  must store a copy  of the

original file somewhere because it will serve  for the transfer of

future correction-file requests.

Split menu

^^^^^^^^^^

Once split menu is entered, it will be possible to choose the file

to be sent.

First,  you must  "select"  the  DIRECTORY   in  which  file to be

transferred is stored (TAB key or click-on the mouse), then choose

the file thath will be marked in inverse-mode character)  and,  at

this point, hit "ALT+O" keys or click-on the [OK].

Fill-in the NAME of the remote addressee: this may be a CALL-SIGN,

or a conventional name such as: TPK, ALL, SOFT, SPLIT,  and so on.

Fill-in the  CALL-SIGN of addresse's  BBS,  or a conventional name

such as ITA, EU, WW.

Fill-in the type of message, namely,  if you send a file-programme

directly to a  given  addressee,  the nature of message will be of

the kind of personal mail, and so, will be qualified  PERSONAL  or

"P".  If  instead,  the  transfer  is intended to be  addressed to

several stations,  that  message will be  qualified as BULLETIN or

"B".  In  the  event that you  are  concerned with the transfer of

splitted-files qualified as BULLETIN, 7PSEND will ask you to enter

a brief  description  of the  file-programme  and  so  besides the

file-programme,  will be sent also a small file that will serve to

describe your file-programme once reaches one's BBS destination.

For a  FBB's BBS, you can read your notes (provided that his SysOp

have installed this option-programme), by hitting "YI".

In the event that, something is got wrong on entering the

requested informations, don't worry,  because you must continue to

enter the  remaining  fields with letters or  words without  sense

until  you reach at a given  menu that  enables you to start again

entering the required informations without destroying anything.

NOTES on .INF files

~~~~~~~~~~~~~~~~~~~

At the risk of being repetitious,  we MUST state  again the  great

importance on using .INF files.

Since here in Italy,  we adopted, starting from some  years ago, a

good   standardized  practice  to  send  together with  split-file

programmes a  .INF  file which serves to explain the contents of a

given file-programme, there is, in this context, a  need to  fully

stress this practice,  and since several  OM abroad from Italy are

now concerned with the use of our programmes.

Practically speaking, when you are using programmes such as 7psTpk

7PSEND (the present programme)  or  SPLITE.EXE,  for  sending your

programmes, a  .INF file accompany  your  split-programme, so when

your split-files  join the  BBS  facility,  servers of the kind of

AUTS24.EXE  etc.  treat  (decode)  also  the  .INF  file  and  its

information is put near the decoded file-programme.

If,  for example,  you are sending the programme  EXAMPLE.EXE, and

put as .INF description:  "This is an example of .INF file",  when

you will connect that  BBS and you'll list its files by giving the

command YI" or "YN" you may read:

.........

EXAMPLE.EXE   171148 24-05-1994    This is an example of .INF file

.........

You may convene that this good practice saves  time  of SysOps and

does a good service for all amateur community!

Send .COR menu

^^^^^^^^^^^^^^

This choose start-up the searching into the  MSG_PRIV list; if any

correction request are live, these are automatically compiled.

WARNING!!!

when correction requests reach your  station,  please  DON'T  READ

THEM!!!  because  in  the  event  you  read  those  messages  they

immediatly  change  themselves  status  from  "P"  to  "Y";  it is

therefore necessary that 7PSEND finds "P" attribute.

Automatically 7PSEND will issue an "intelligent" reply, that is it

will send a .COR file-message to the CALL-SIGN, AUTO7P  or  7PSERV

and so on, in order to satisfy the received request.

END menu

^^^^^^^^

It is the word itself that announce that process is getting to the

END; now we can run TPK and connect our local BBS  to transfer the

programme.

As for  final  7PSEND's  system  setup,  I am sure  that you  have

already activated the 7PLUS DECODE function of TPK programme.

So  you can easely decode splits reaching one after the other when

you download them from your local BBS and, if  everything has been

right, you can get a "complete" file once the l ast split-file has

been downloaded and decoded.

So, if instead some error occours,  TPK automatically  generates a

file-message  for correction request,  which will be imported into

the MSG_PRIV list to be forwarded.

That message-file will be addressed to your call-sign, so later on

you must only re-address that  message towards  the  originator of

the file which has not been rebuilt.

NOTES OF THE AUTHOR

^^^^^^^^^^^^^^^^^^^

The use  of  this  programme  is  completely  free of charge,  but

however any form of its commercialization remain not authorized.

This programme took me time and efforts so if you install 7PSEND I

would like to know your name together with your own  impression(s)

by sending to me a retourn message.

Version 2.0 modifications

^^^^^^^^^^^^^^^^^^^^^^^^^

This version  of  the programme  has   been submitted to a general

optimization.  Some  minor  bugs  had  been  eliminated  thanks to

indications of the previous users.

At last I replaced the label "7PSTpk:" with "7+Send:"  in order to

preclude at anyone the possibility to  "capture",  by means of the

function  BR STRING  of TPK programme,  all messages containing in

their names the word "TPK" and so enable to import all split-files

by this programme.

APPENDIX NOTES: The  original  documentation  in italian  language  was

edited by Claudio IW0DOP.

English translation edited by Gustavo I0OJJ.

Comment(s) or suggestion(s): IW0DOP@I0VNR.ILAZ.ITA.EU